Power Tool Inspection Checklist

DEFECTIVE TOOLS MUST NOT BE USED. Remove from service, tag as defective, and report to supervisor.

Pre-use visual inspection for electric, battery, and pneumatic power tools in accordance with PUWER 1998 and the Electricity at Work Regulations 1989

| Inspection Reference: | PTI- |
| --- | --- |
| Date: | DD/MM/YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Tool Type: | e.g., Angle grinder, SDS drill, circular saw, reciprocating saw, jigsaw, planer, router |
| Tool Make / Model: |  |
| Serial / Asset Number: |  |
| Power Source: | 110V Electric / 240V Electric / Battery / Pneumatic |
| PAT Test Due Date: | DD/MM/YYYY (electric tools only) |
| Owner: | PC / SC / Hire (name) |
| Inspected By: | Name and role |

Section A: Electrical Checks (Corded Electric Tools)

| No. | Electrical Check | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | PAT test in date (label attached showing test date and next due date) |  |  |  |  |
| 2 | Supply cable: no cuts, nicks, abrasion, kinks, or exposed conductors |  |  |  |  |
| 3 | Plug: no cracks, pins straight and secure, cable grip secure with no bare wires visible |  |  |  |  |
| 4 | Cable entry to tool: secure, no strain on connections, cable grip intact |  |  |  |  |
| 5 | On/off switch: functioning correctly, returns to off position when released (dead-man switch where fitted) |  |  |  |  |
| 6 | Tool casing: no cracks, damage, or missing screws/covers |  |  |  |  |
| 7 | Ventilation slots: clear and unobstructed |  |  |  |  |
| 8 | 110V centre-tapped earth system used on site (yellow plugs and sockets) |  |  |  |  |
| 9 | Transformer (110V): in good condition, PAT tested, and correctly rated for the tool |  |  |  |  |

Section B: Battery Tool Checks

| No. | Battery Check | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Battery and charger in good condition with no visible damage, swelling, or leakage |  |  |  |  |
| 2 | Battery securely seated in the tool with latch engaged |  |  |  |  |
| 3 | Charger cable and plug in good condition |  |  |  |  |
| 4 | Battery charged and holding charge adequately |  |  |  |  |
| 5 | Battery type and voltage correct for the tool (as per manufacturer specification) |  |  |  |  |

Section C: Mechanical and Guard Checks

| No. | Mechanical / Guard Check | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Guards fitted, secure, and functioning correctly (blade guards, disc guards, chuck guards) |  |  |  |  |
| 2 | Guard not modified, removed, or tied back |  |  |  |  |
| 3 | Cutting disc / blade / bit: correct type and size for the tool and task, in good condition, no cracks or chips |  |  |  |  |
| 4 | Disc / blade securely fitted using the correct locking mechanism |  |  |  |  |
| 5 | Chuck key removed after tightening (where applicable) |  |  |  |  |
| 6 | Side handle / auxiliary handle fitted and secure (where provided by manufacturer) |  |  |  |  |
| 7 | Depth gauge / fence / guide correctly adjusted |  |  |  |  |
| 8 | Dust extraction port clear and connected to extraction system where required |  |  |  |  |
| 9 | Anti-vibration features functioning (where fitted) |  |  |  |  |

Section D: General Safety Checks

| No. | General Safety Check | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Operator trained and competent to use this specific tool type |  |  |  |  |
| 2 | Operator wearing correct PPE for the tool: eye protection, hearing protection, gloves, dust mask as required |  |  |  |  |
| 3 | Tool used in accordance with manufacturer instructions and site RAMS |  |  |  |  |
| 4 | Work area clean and clear; workpiece secured / clamped before cutting or drilling |  |  |  |  |
| 5 | No loose clothing, jewellery, or untied hair that could become entangled |  |  |  |  |
| 6 | Tool stored correctly after use: disconnected from power, blade retracted or removed, stored in case or designated rack |  |  |  |  |

Inspection Outcome

| Outcome | Tick |
| --- | --- |
| FIT FOR USE — All checks passed. |  |
| NOT FIT FOR USE — Defect identified. Tool removed from service. |  |

Defects Identified

| No. | Defect Description | Action Taken | Priority | Reported To |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |

Inspector Declaration

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Inspected By |  |  | DD/MM/YYYY |
| Supervisor (if defect) |  |  | DD/MM/YYYY |

Document Control

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 0 | DD/MM/YYYY |  | Initial issue |