Window and Door Installation

Inspection Checklist

| Document Ref: | Enter reference | Revision: | Enter revision |
| --- | --- | --- | --- |
| Project Name: | Enter project name | Contract No: | Enter contract number |
| Date: | DD / MM / YYYY | Inspected By: | Name and role |
| Approved By: | Name and role | Plot / Building: | Enter details |

# 1. Unit Details

| Unit Reference: | Enter window/door reference from schedule |
| --- | --- |
| Type: | e.g. Casement window, sliding door, entrance door, roof window, bi-fold |
| Material: | e.g. uPVC, aluminium, timber, composite, steel |
| Manufacturer: | Enter manufacturer name |
| Size (W x H): | mm x mm |
| Glazing Specification: | e.g. Double glazed, triple glazed, U-value, Pilkington K Glass |
| U-Value: | W/m²K (whole unit) |
| Fire Rating (if applicable): | e.g. FD30, FD60, or N/A |
| Drawing Reference: | Enter elevation and detail drawing refs |

# 2. Installation Inspection

| No. | Check Item | Pass | Fail | N/A | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Opening dimensions checked before installation (width, height, squareness) |  |  |  |  |
| 2 | Lintel installed and correctly bearing at both ends |  |  |  |  |
| 3 | Cavity tray / DPC above opening installed correctly with stop ends |  |  |  |  |
| 4 | Weep holes provided above lintel at maximum 450mm centres |  |  |  |  |
| 5 | Frame plumb, level, and square (checked with spirit level) |  |  |  |  |
| 6 | Frame fixed with correct fixings at specified centres (max 150mm from corners, 600mm between) |  |  |  |  |
| 7 | Packers used to support frame – not relying on expanding foam alone |  |  |  |  |
| 8 | Cavity closer installed and continuous around the frame |  |  |  |  |
| 9 | Expanding foam applied as supplementary seal (not primary fixing or seal) |  |  |  |  |
| 10 | Internal plaster / render stop bead fitted around frame |  |  |  |  |
| 11 | External sealant applied (mastic) – correct product, continuous bead, tooled finish |  |  |  |  |
| 12 | Internal vapour barrier / membrane sealed to frame perimeter |  |  |  |  |
| 13 | Trickle ventilator installed in correct position (if specified) |  |  |  |  |
| 14 | Hardware (handles, locks, restrictors) fitted and operating correctly |  |  |  |  |
| 15 | Window / door opens, closes, and latches correctly without binding |  |  |  |  |
| 16 | Drainage slots / weep holes in frame clear and unobstructed |  |  |  |  |
| 17 | Glazing sealed correctly with no gaps or broken seals |  |  |  |  |
| 18 | Threshold / cill installed with correct fall to shed water |  |  |  |  |
| 19 | Fire door: intumescent strip and smoke seal installed (if FD rated) |  |  |  |  |
| 20 | Fire door: self-closer fitted and operating correctly (if required) |  |  |  |  |

# 3. Sign-Off

| Installer / Contractor |  | Site Manager / Inspector |  |
| --- | --- | --- | --- |
| Name: | Print name | Name: | Print name |
| Signature: |  | Signature: |  |
| Date: | DD / MM / YYYY | Date: | DD / MM / YYYY |

# 4. Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| 01 | DD/MM/YYYY | Initial issue |  |  |
|  | DD/MM/YYYY |  |  |  |
|  | DD/MM/YYYY |  |  |  |