Damp Proof Course Installation

Record

| Document Ref: | Enter reference | Revision: | Enter revision |
| --- | --- | --- | --- |
| Project Name: | Enter project name | Contract No: | Enter contract number |
| Date: | DD / MM / YYYY | Installed By: | Name and role |
| Approved By: | Name and role | Inspected By: | Enter details |

# 1. Location Details

| Building / Plot: | Enter building or plot reference |
| --- | --- |
| Location: | Enter wall reference, gridline, or elevation |
| DPC Level (mAOD): | Enter level – minimum 150mm above finished ground level |
| Drawing Reference: | Enter construction drawing number and revision |
| Specification Clause: | Enter specification or NHBC requirement reference |

# 2. DPC Material Details

| DPC Type: | e.g. Polyethylene, bitumen polymer, pitch polymer, proprietary cavity tray |
| --- | --- |
| DPC Width: | mm (must be full wall width or cavity tray width) |
| Manufacturer: | Enter manufacturer name |
| Product Reference: | Enter product name and reference |
| Batch / Roll Number: | Enter for traceability |
| BBA / BRE Certificate? | Yes / No – enter certificate number |

# 3. Installation Checklist

| No. | Check Item | Pass | Fail | N/A | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | DPC laid on full bed of mortar – no voids or gaps beneath |  |  |  |  |
| 2 | DPC extends full width of the wall (not cut short) |  |  |  |  |
| 3 | DPC minimum 150mm above finished external ground level |  |  |  |  |
| 4 | Laps at joints minimum 100mm and sealed where required |  |  |  |  |
| 5 | DPC corners and angles formed correctly without tearing |  |  |  |  |
| 6 | DPC not punctured, torn, or damaged during installation |  |  |  |  |
| 7 | Cavity tray DPC installed with correct fall towards outer leaf |  |  |  |  |
| 8 | Cavity tray stop ends and weep holes installed at correct centres |  |  |  |  |
| 9 | DPC stepped correctly at changes in level (stepped foundation) |  |  |  |  |
| 10 | DPC below timber wall plates to protect from rising damp |  |  |  |  |
| 11 | DPC linked to DPM (damp proof membrane) where they meet |  |  |  |  |
| 12 | DPC at door and window thresholds installed and lapped correctly |  |  |  |  |
| 13 | Weep holes clear and open above cavity tray DPC |  |  |  |  |
| 14 | No mortar droppings bridging the DPC in the cavity |  |  |  |  |

# 4. Sign-Off

| Mason / Installer |  | Engineer / Inspector |  |
| --- | --- | --- | --- |
| Name: | Print name | Name: | Print name |
| Signature: |  | Signature: |  |
| Date: | DD / MM / YYYY | Date: | DD / MM / YYYY |

# 5. Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| 01 | DD/MM/YYYY | Initial issue |  |  |
|  | DD/MM/YYYY |  |  |  |
|  | DD/MM/YYYY |  |  |  |