Concrete Repair Specification

Record

| Document Ref: | Enter reference | Revision: | Enter revision |
| --- | --- | --- | --- |
| Project Name: | Enter project name | Contract No: | Enter contract number |
| Date: | DD / MM / YYYY | Prepared By: | Name and role |
| Approved By: | Name and role | Repair No: | Enter sequential repair number |

# 1. Defect Identification

| Structure / Element: | e.g. Retaining wall, bridge abutment, water-retaining structure, floor slab |
| --- | --- |
| Location / Gridline: | Enter precise location of defect |
| Drawing Reference: | Enter structural drawing number and revision |
| Type of Defect: | e.g. Spalling, delamination, cracking, honeycombing, chloride contamination, carbonation |
| Cause of Defect: | e.g. Reinforcement corrosion, freeze-thaw, chemical attack, impact damage, construction defect |
| Depth of Defect: | mm (maximum depth of deterioration behind reinforcement) |
| Area of Repair: | m² (estimated area to be repaired) |
| Reinforcement Exposed? | Yes / No – if Yes, state bar sizes and condition |
| Reinforcement Corrosion? | None / Surface rust / Section loss – state estimated % section loss |
| Structural Engineer Consulted? | Yes / No – name and date |
| Condition Survey Ref: | Enter condition survey or investigation report reference |

# 2. Repair System Specification

| BS EN 1504 Principle: | e.g. Principle 3 (Concrete restoration), Principle 4 (Structural strengthening), Principle 7 (Corrosion protection) |
| --- | --- |
| BS EN 1504 Method: | e.g. Method 3.1 (Hand-applied mortar), Method 3.2 (Recasting with concrete), Method 3.3 (Sprayed concrete) |
| Repair Mortar / Material: | Enter product name, manufacturer, and class (e.g. Class R3, R4) |
| Product Data Sheet Ref: | Enter manufacturer’s TDS reference |
| Bonding Agent / Primer: | Enter product name or state 'Not required' (SBR, acrylic, epoxy) |
| Reinforcement Treatment: | e.g. Anti-corrosion coating, zinc-rich primer, or none required |
| Protective Coating: | e.g. Anti-carbonation coating, silane impregnation, or none |
| Minimum Repair Depth: | mm (minimum cover to reinforcement after repair) |
| Maximum Layer Thickness: | mm per layer (as per manufacturer’s guidance) |
| Curing Requirements: | e.g. Damp cure 24 hrs, curing membrane, polythene cover |

# 3. Repair Procedure Checklist

| No. | Check Item | Pass | Fail | N/A | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Defect area marked out and approved by engineer before preparation |  |  |  |  |
| 2 | Concrete broken out to sound material (min 10mm behind reinforcement) |  |  |  |  |
| 3 | Edges of repair area cut square (no feather edges) |  |  |  |  |
| 4 | Exposed reinforcement cleaned to bright metal (Sa 2.5 or equivalent) |  |  |  |  |
| 5 | Section loss in reinforcement assessed – supplementary bars added if > 20% |  |  |  |  |
| 6 | Substrate dampened (saturated surface dry) before mortar application |  |  |  |  |
| 7 | Bonding agent / primer applied as per manufacturer’s instructions |  |  |  |  |
| 8 | Repair mortar mixed as per manufacturer’s instructions (water ratio recorded) |  |  |  |  |
| 9 | Mortar applied in layers not exceeding manufacturer’s maximum thickness |  |  |  |  |
| 10 | Each layer keyed for next layer (if multi-layer application) |  |  |  |  |
| 11 | Final surface finished to match surrounding concrete profile |  |  |  |  |
| 12 | Curing commenced immediately after finishing |  |  |  |  |
| 13 | Protective coating applied after curing period (if specified) |  |  |  |  |
| 14 | Repair photographed before, during, and after completion |  |  |  |  |

# 4. Repair Log

| No. | Repair Location | Area m² | Depth mm | Material Used | Batch No. | Date Completed |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  | m² | mm |  |  | DD/MM/YYYY |
| 2 |  | m² | mm |  |  | DD/MM/YYYY |
| 3 |  | m² | mm |  |  | DD/MM/YYYY |
| 4 |  | m² | mm |  |  | DD/MM/YYYY |
| 5 |  | m² | mm |  |  | DD/MM/YYYY |
| 6 |  | m² | mm |  |  | DD/MM/YYYY |
| 7 |  | m² | mm |  |  | DD/MM/YYYY |
| 8 |  | m² | mm |  |  | DD/MM/YYYY |
| 9 |  | m² | mm |  |  | DD/MM/YYYY |
| 10 |  | m² | mm |  |  | DD/MM/YYYY |

# 5. Sign-Off

| Repair Contractor |  | Engineer / Inspector |  |
| --- | --- | --- | --- |
| Name: | Print name | Name: | Print name |
| Company: | Enter company | Position: | Enter role |
| Signature: |  | Signature: |  |
| Date: | DD / MM / YYYY | Date: | DD / MM / YYYY |

# 6. Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| 01 | DD/MM/YYYY | Initial issue |  |  |
|  | DD/MM/YYYY |  |  |  |
|  | DD/MM/YYYY |  |  |  |