Manhole Shutter and Pour

Record

| Document Ref: | Enter reference | Revision: | Enter revision |
| --- | --- | --- | --- |
| Project Name: | Enter project name | Contract No: | Enter contract number |
| Date of Pour: | DD / MM / YYYY | Recorded By: | Name and role |
| Approved By: | Name and role | Pour Ref No: | Enter pour reference |

# 1. Manhole Details

| Manhole Reference: | Enter MH reference from drawings (e.g. MH-101) |
| --- | --- |
| Location / Chainage: | Enter gridline, chainage, or plot reference |
| Manhole Type: | e.g. Circular, rectangular, backdrop, drop connection |
| Internal Dimensions: | Diameter mm or Length x Width mm |
| Depth (GL to Invert): | metres |
| Invert Level (mAOD): | mAOD |
| Cover Level (mAOD): | mAOD |
| Wall Thickness: | mm |
| Base Slab Thickness: | mm |
| Concrete Specification: | e.g. C32/40, RC35/45, with or without WRA, designated mix |
| Structural Drawing Ref: | Enter drawing number and revision |
| Shutter System: | e.g. Steel circular shutter, timber, proprietary (state manufacturer) |

# 2. Pre-Pour Checklist

| No. | Check Item | Pass | Fail | N/A | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Excavation base prepared, level, and compacted (blinding concrete in place if specified) |  |  |  |  |
| 2 | Formation inspection completed and signed off |  |  |  |  |
| 3 | Reinforcement installed and inspected per structural drawings |  |  |  |  |
| 4 | Rebar inspection request signed off before shutter erected |  |  |  |  |
| 5 | Shutter erected plumb, level, and at correct dimensions |  |  |  |  |
| 6 | Shutter clean, release agent applied to all faces |  |  |  |  |
| 7 | Shutter braced and secured against concrete pressure |  |  |  |  |
| 8 | Waterstops installed at construction joints (if specified) |  |  |  |  |
| 9 | Pipe entries correctly positioned: size, invert level, and angle verified |  |  |  |  |
| 10 | Pipe entry seals (puddle flanges, link seals) in correct position |  |  |  |  |
| 11 | Benching formwork positioned at correct falls (1:12 minimum) |  |  |  |  |
| 12 | Step irons or ladder positioned at correct centres (if specified) |  |  |  |  |
| 13 | Cover frame set at correct level and orientation (if cast in) |  |  |  |  |
| 14 | Channel inverts formed at correct gradient matching pipe inverts |  |  |  |  |
| 15 | Spacers and cover blocks positioned to achieve minimum cover |  |  |  |  |
| 16 | Concrete supply confirmed: delivery tickets, mix design, volume required |  |  |  |  |
| 17 | Weather conditions assessed and acceptable for pour |  |  |  |  |

# 3. Concrete Delivery Record

| Load | Ticket No. | Arrive | Pour | Vol m³ | Slump mm | Temp °C | Accept? | Cubes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | HH:MM | HH:MM | m³ | mm | °C | Y/N | Y/N |
| 2 |  | HH:MM | HH:MM | m³ | mm | °C | Y/N | Y/N |
| 3 |  | HH:MM | HH:MM | m³ | mm | °C | Y/N | Y/N |
| 4 |  | HH:MM | HH:MM | m³ | mm | °C | Y/N | Y/N |
| 5 |  | HH:MM | HH:MM | m³ | mm | °C | Y/N | Y/N |
| 6 |  | HH:MM | HH:MM | m³ | mm | °C | Y/N | Y/N |

| Total Volume Placed: | m³ |
| --- | --- |
| Pour Start Time: | HH:MM |
| Pour Finish Time: | HH:MM |

# 4. Post-Pour / Striking

| Curing Method: | e.g. Curing compound, polythene, wet hessian, leave in shutter |
| --- | --- |
| Minimum Curing Period: | hours / days (as specified) |
| Striking Date: | DD / MM / YYYY |
| Surface Finish Quality: | Describe: blowholes, honeycombing, surface defects, or satisfactory |
| Defects Found? | Yes / No – if Yes, describe and state repair method |
| Backfill Authorised? | Yes / No – name and date |

# 5. Sign-Off

| Contractor / Foreman |  | Engineer / Inspector |  |
| --- | --- | --- | --- |
| Name: | Print name | Name: | Print name |
| Signature: |  | Signature: |  |
| Date: | DD / MM / YYYY | Date: | DD / MM / YYYY |

# 6. Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| 01 | DD/MM/YYYY | Initial issue |  |  |
|  | DD/MM/YYYY |  |  |  |
|  | DD/MM/YYYY |  |  |  |