Water Main Disinfection and

Sampling Record

| Document Ref: | Enter reference | Revision: | Enter revision |
| --- | --- | --- | --- |
| Project Name: | Enter project name | Contract No: | Enter contract number |
| Date: | DD / MM / YYYY | Carried Out By: | Name and company |
| Witnessed By: | Name and role | Approved By: | Name and role |

# 1. Pipeline Details

| Main Reference: | Enter pipeline reference |
| --- | --- |
| Section (From – To): | Enter start and end points |
| Pipe Material: | e.g. Ductile iron, PE100, uPVC |
| Nominal Diameter (DN): | mm |
| Length: | metres |
| Volume: | litres |
| Water Company: | Enter water company name |
| Pressure Test Cert Ref: | Enter reference |
| Applicable Standard: | Water UK / BS EN 805 / local water company specification |

# 2. Pre-Disinfection Checklist

| No. | Check Item | Yes | No | N/A | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Pipeline pressure tested and passed |  |  |  |  |
| 2 | Pipeline flushed with potable water until visually clear |  |  |  |  |
| 3 | Swabbing completed (foam swab passed through) if required |  |  |  |  |
| 4 | All connections valved off or capped |  |  |  |  |
| 5 | COSHH assessment for chlorine products completed and briefed |  |  |  |  |
| 6 | PPE available: goggles, face shield, chemical gloves |  |  |  |  |
| 7 | Emergency eye wash and spill kit at dosing point |  |  |  |  |
| 8 | Dechlorination agent available (sodium thiosulphate) |  |  |  |  |
| 9 | Discharge route and consent confirmed |  |  |  |  |
| 10 | Water company notified and approval received |  |  |  |  |
| 11 | Sampling plan agreed with water company |  |  |  |  |
| 12 | UKAS/MCERTS accredited laboratory arranged for bacteriological analysis |  |  |  |  |

# 3. Disinfection Details

| Disinfection Method: | e.g. Slug dose, continuous feed, tablet, static |
| --- | --- |
| Chlorine Source: | e.g. Sodium hypochlorite 14%, calcium hypochlorite tablets |
| Target Concentration: | mg/l free chlorine |
| Contact Period Required: | hours (typically 16–24 hours) |
| Dosing Start: | DD/MM/YYYY HH:MM |
| Dosing End: | DD/MM/YYYY HH:MM |
| Contact Period Start: | DD/MM/YYYY HH:MM |
| Contact Period End: | DD/MM/YYYY HH:MM |
| Actual Contact Duration: | hours |

# 4. Chlorine Residual Monitoring

| No. | Time | Sample Location | Free Cl mg/l | Total Cl mg/l | Min Req mg/l | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | HH:MM |  | mg/l | mg/l | mg/l |  |
| 2 | HH:MM |  | mg/l | mg/l | mg/l |  |
| 3 | HH:MM |  | mg/l | mg/l | mg/l |  |
| 4 | HH:MM |  | mg/l | mg/l | mg/l |  |
| 5 | HH:MM |  | mg/l | mg/l | mg/l |  |
| 6 | HH:MM |  | mg/l | mg/l | mg/l |  |
| 7 | HH:MM |  | mg/l | mg/l | mg/l |  |
| 8 | HH:MM |  | mg/l | mg/l | mg/l |  |
| 9 | HH:MM |  | mg/l | mg/l | mg/l |  |
| 10 | HH:MM |  | mg/l | mg/l | mg/l |  |

# 5. Dechlorination and Flushing

| Dechlorination Agent: | e.g. Sodium thiosulphate |
| --- | --- |
| Volume Used: | litres or kg |
| Flushing Start: | DD/MM/YYYY HH:MM |
| Flushing End: | DD/MM/YYYY HH:MM |
| Residual Cl After Flush: | mg/l |
| Discharge Point: | e.g. Foul sewer, tanker, watercourse (dechlorinated) |

# 6. Bacteriological Sampling Results

| No. | Sample Location | Date/Time | Coliforms | E. coli | Free Cl | Turbidity | Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | DD/MM HH:MM | cfu/100ml | cfu/100ml | mg/l | NTU | Pass/Fail |
| 2 |  | DD/MM HH:MM | cfu/100ml | cfu/100ml | mg/l | NTU | Pass/Fail |
| 3 |  | DD/MM HH:MM | cfu/100ml | cfu/100ml | mg/l | NTU | Pass/Fail |
| 4 |  | DD/MM HH:MM | cfu/100ml | cfu/100ml | mg/l | NTU | Pass/Fail |
| 5 |  | DD/MM HH:MM | cfu/100ml | cfu/100ml | mg/l | NTU | Pass/Fail |
| 6 |  | DD/MM HH:MM | cfu/100ml | cfu/100ml | mg/l | NTU | Pass/Fail |
| 7 |  | DD/MM HH:MM | cfu/100ml | cfu/100ml | mg/l | NTU | Pass/Fail |
| 8 |  | DD/MM HH:MM | cfu/100ml | cfu/100ml | mg/l | NTU | Pass/Fail |

Pass criteria: Coliforms = 0 cfu/100ml, E. coli = 0 cfu/100ml in consecutive samples ≥24 hours apart.

# 7. Overall Result

| RESULT: | PASS – Main Fit for Service / FAIL – Re-disinfection Required |
| --- | --- |
| Consecutive Clear Samples? | Yes / No |
| Connection Authorised? | Yes / No – water company authorisation ref |
| Connection Date: | DD / MM / YYYY |

# 8. Sign-Off

| Contractor |  | Water Company / Client |  |
| --- | --- | --- | --- |
| Name: | Print name | Name: | Print name |
| Signature: |  | Signature: |  |
| Date: | DD / MM / YYYY | Date: | DD / MM / YYYY |

# 9. Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| 01 | DD/MM/YYYY | Initial issue |  |  |
|  | DD/MM/YYYY |  |  |  |
|  | DD/MM/YYYY |  |  |  |