Cold Weather Concreting Procedure

| Document Ref: | Enter reference | Revision: | Enter revision |
| --- | --- | --- | --- |
| Project Name: | Enter project name | Contract No: | Enter contract number |
| Date: | DD / MM / YYYY | Prepared By: | Name and role |
| Approved By: | Name and role | Season / Period: | e.g. October – March |

# 1. Purpose and Scope

This procedure sets out the requirements for placing and curing concrete during cold weather conditions on this project. It applies when the ambient air temperature is at or below 5°C, or when the temperature is forecast to fall below 0°C within 24 hours of placing. This procedure should be read alongside BS EN 206, BS 8500, and CIRIA Report C660. Amend guidance text and delete before issuing.

# 2. Temperature Thresholds and Actions

| Temperature | Required Action |
| --- | --- |
| Above 5°C | Normal concreting procedures apply. Monitor forecast. |
| 2°C to 5°C | Cold weather precautions activated. Implement this procedure in full. Protect concrete after placing. |
| 0°C to 2°C | Enhanced protection required. Heated enclosures or insulating blankets mandatory. Extend curing period. |
| Below 0°C | Do not place concrete unless approved by the structural engineer and enhanced heated protection is confirmed. Risk of frost damage to fresh concrete. |

# 3. Pre-Pour Checklist

| No. | Procedure Item | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Weather forecast checked for the next 72 hours (min/max temperatures) |  |  |  |  |
| 2 | Ambient temperature recorded at time of pour decision |  |  |  |  |
| 3 | Ground / substrate temperature checked (must be above 0°C) |  |  |  |  |
| 4 | All ice, snow, and frost removed from formwork, reinforcement, and substrate |  |  |  |  |
| 5 | Formwork heated or insulated prior to pour (if temperature < 2°C) |  |  |  |  |
| 6 | Reinforcement free from ice and snow at time of placing |  |  |  |  |
| 7 | Concrete supplier notified of cold weather conditions and delivery requirements |  |  |  |  |
| 8 | Concrete delivery temperature specified (minimum 10°C at point of discharge) |  |  |  |  |
| 9 | Concrete mix design reviewed for cold weather (consider increased cement content or admixtures) |  |  |  |  |
| 10 | Accelerating admixture approved and dosage confirmed (if used) |  |  |  |  |
| 11 | Hot water in mix to raise concrete temperature confirmed with supplier |  |  |  |  |
| 12 | Insulating blankets, thermal quilts, or heated enclosures available on site |  |  |  |  |
| 13 | Curing compounds, membrane, or polythene sheeting available |  |  |  |  |
| 14 | Temperature monitoring equipment (thermometer, data logger) calibrated and ready |  |  |  |  |
| 15 | Pour sequence planned to minimise exposure time of fresh concrete |  |  |  |  |
| 16 | Standby heating equipment available (if enclosure heating required) |  |  |  |  |

# 4. During Pour

| No. | Procedure Item | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Concrete temperature checked at point of discharge (record below) |  |  |  |  |
| 2 | Concrete temperature at discharge not below 10°C |  |  |  |  |
| 3 | Concrete placed and compacted rapidly to minimise heat loss |  |  |  |  |
| 4 | No concrete placed on frozen ground or against frozen formwork |  |  |  |  |
| 5 | Pour rate adjusted to account for slower setting in cold weather |  |  |  |  |
| 6 | Concrete not over-vibrated (risk of bleed water in cold conditions) |  |  |  |  |
| 7 | Construction joints planned to avoid cold joints from slow setting |  |  |  |  |

# 5. Post-Pour Protection and Curing

| No. | Procedure Item | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Concrete surfaces covered immediately after finishing |  |  |  |  |
| 2 | Insulating blankets or thermal quilts applied to exposed surfaces |  |  |  |  |
| 3 | Edges and corners given additional protection (highest heat loss areas) |  |  |  |  |
| 4 | Heated enclosure erected and maintained (if temperature < 2°C) |  |  |  |  |
| 5 | Minimum concrete temperature maintained at 5°C for initial curing period |  |  |  |  |
| 6 | Curing period extended by 50% or as directed by structural engineer |  |  |  |  |
| 7 | Temperature monitoring commenced immediately after placing |  |  |  |  |
| 8 | Temperature differential between core and surface monitored (max 20°C) |  |  |  |  |
| 9 | Formwork not struck until concrete achieves required strength |  |  |  |  |
| 10 | Striking criteria agreed with structural engineer for cold weather conditions |  |  |  |  |
| 11 | Protection maintained until concrete achieves minimum 5 N/mm² compressive strength |  |  |  |  |

# 6. Temperature Monitoring Record

| Date | Time | Ambient °C | Concrete °C | Surface °C | Core °C | Action Taken |
| --- | --- | --- | --- | --- | --- | --- |
| DD/MM | HH:MM | °C | °C | °C | °C |  |
| DD/MM | HH:MM | °C | °C | °C | °C |  |
| DD/MM | HH:MM | °C | °C | °C | °C |  |
| DD/MM | HH:MM | °C | °C | °C | °C |  |
| DD/MM | HH:MM | °C | °C | °C | °C |  |
| DD/MM | HH:MM | °C | °C | °C | °C |  |
| DD/MM | HH:MM | °C | °C | °C | °C |  |
| DD/MM | HH:MM | °C | °C | °C | °C |  |
| DD/MM | HH:MM | °C | °C | °C | °C |  |
| DD/MM | HH:MM | °C | °C | °C | °C |  |
| DD/MM | HH:MM | °C | °C | °C | °C |  |
| DD/MM | HH:MM | °C | °C | °C | °C |  |

# 7. Concrete Delivery Record

| Load No. | Ticket No. | Time Arrive | Time Pour | Temp °C | Slump mm | Vol m³ | Accepted? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | HH:MM | HH:MM | °C | mm | m³ | Y/N |
| 2 |  | HH:MM | HH:MM | °C | mm | m³ | Y/N |
| 3 |  | HH:MM | HH:MM | °C | mm | m³ | Y/N |
| 4 |  | HH:MM | HH:MM | °C | mm | m³ | Y/N |
| 5 |  | HH:MM | HH:MM | °C | mm | m³ | Y/N |
| 6 |  | HH:MM | HH:MM | °C | mm | m³ | Y/N |
| 7 |  | HH:MM | HH:MM | °C | mm | m³ | Y/N |
| 8 |  | HH:MM | HH:MM | °C | mm | m³ | Y/N |

# 8. Sign-Off

| Prepared By: | Name | Signature: |  |
| --- | --- | --- | --- |
| Position / Role: | Enter role | Date: | DD / MM / YYYY |
| Approved By: | Name | Signature: |  |
| Date: | DD / MM / YYYY | Next Review: | DD / MM / YYYY |

# 9. Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| 01 | DD/MM/YYYY | Initial issue |  |  |
|  | DD/MM/YYYY |  |  |  |
|  | DD/MM/YYYY |  |  |  |