Self-Compacting Concrete

Placement Record

| Document Ref: | Enter reference | Revision: | Enter revision |
| --- | --- | --- | --- |
| Project Name: | Enter project name | Contract No: | Enter contract number |
| Date of Pour: | DD / MM / YYYY | Pour Ref No: | Enter pour reference |
| Prepared By: | Name and role | Approved By: | Name and role |

# 1. Element and Location

| Structural Element: | e.g. Secant pile, diaphragm wall, base slab, column, complex geometry |
| --- | --- |
| Location / Gridlines: | Enter gridline references and level |
| Structural Drawing Ref: | Enter drawing number and revision |
| Pour Volume (designed): | m³ |
| Reason for SCC: | e.g. Congested reinforcement, complex geometry, visual concrete, noise restriction |

# 2. SCC Mix Design

| Concrete Supplier: | Enter supplier name and plant |
| --- | --- |
| Mix Design Ref: | Enter mix design reference number |
| Strength Class: | e.g. C32/40, C40/50, C50/60 |
| Exposure Class: | e.g. XC2, XC3/4, XD1, XS1 |
| Max Aggregate Size: | e.g. 10mm, 14mm, 20mm |
| Cement / Binder Type: | e.g. CEM I, CEM III/A, PFA, GGBS, silica fume |
| Admixtures: | e.g. Superplasticiser, VMA (viscosity modifying agent), retarder |
| Slump Flow Class: | e.g. SF1 (550-650mm), SF2 (660-750mm), SF3 (760-850mm) |
| Viscosity Class: | e.g. VS1, VS2 (T500 test), VF1, VF2 (V-funnel test) |
| Passing Ability Class: | e.g. PL1, PL2 (L-box), PJ1, PJ2 (J-ring) |
| Segregation Resistance: | e.g. SR1, SR2 (sieve segregation test) |

# 3. Pre-Pour Checklist

| No. | Check Item | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | SCC mix design approved by structural engineer |  |  |  |  |
| 2 | Trial mix or mock-up pour completed and results acceptable |  |  |  |  |
| 3 | Formwork watertight – all joints sealed to prevent grout loss |  |  |  |  |
| 4 | Formwork designed for SCC fluid pressure (higher than conventional concrete) |  |  |  |  |
| 5 | Formwork ties and bracing checked for SCC loading |  |  |  |  |
| 6 | Reinforcement inspection complete and approved |  |  |  |  |
| 7 | Pour sequence and filling points agreed and communicated |  |  |  |  |
| 8 | Tremie or pump pipe positions planned to avoid free fall > 0.5m |  |  |  |  |
| 9 | Concrete supplier briefed on SCC requirements and testing regime |  |  |  |  |
| 10 | Testing equipment on site: slump flow plate, T500 timer, L-box, V-funnel |  |  |  |  |
| 11 | Cube moulds prepared (do not vibrate SCC cubes) |  |  |  |  |
| 12 | Weather forecast checked – temperature and rain |  |  |  |  |
| 13 | Operatives briefed: no vibration of SCC unless specifically agreed |  |  |  |  |

# 4. Delivery and Testing Record

| Load | Ticket | Time | Vol m³ | Temp °C | Slump Flow mm | T500 sec | V-Funnel sec | Accept? | Cubes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | HH:MM |  | °C | mm | s | s | Y/N | Y/N |
| 2 |  | HH:MM |  | °C | mm | s | s | Y/N | Y/N |
| 3 |  | HH:MM |  | °C | mm | s | s | Y/N | Y/N |
| 4 |  | HH:MM |  | °C | mm | s | s | Y/N | Y/N |
| 5 |  | HH:MM |  | °C | mm | s | s | Y/N | Y/N |
| 6 |  | HH:MM |  | °C | mm | s | s | Y/N | Y/N |
| 7 |  | HH:MM |  | °C | mm | s | s | Y/N | Y/N |
| 8 |  | HH:MM |  | °C | mm | s | s | Y/N | Y/N |
| 9 |  | HH:MM |  | °C | mm | s | s | Y/N | Y/N |
| 10 |  | HH:MM |  | °C | mm | s | s | Y/N | Y/N |

| Total Volume Delivered: | m³ |
| --- | --- |
| Total Volume Placed: | m³ |
| Pour Start Time: | HH:MM |
| Pour Finish Time: | HH:MM |
| Total Pour Duration: | hours |

# 5. Placement Observations

Record observations during the pour: flow behaviour, filling of formwork, any signs of segregation, blocking, air entrapment, or surface defects. Note any loads rejected and reasons.

# 6. Curing and Striking

| Curing Method: | e.g. Curing compound, polythene, wet hessian, formwork left in place |
| --- | --- |
| Curing Commenced: | Time and date |
| Minimum Curing Period: | e.g. 3 days, 7 days, as specified |
| Striking Criteria: | e.g. 10 N/mm², 15 N/mm², or as directed by structural engineer |
| Date Struck: | DD / MM / YYYY |
| Surface Finish Quality: | Describe: blowholes, surface finish class, colour uniformity |

# 7. Sign-Off

| Contractor |  | Engineer / Inspector |  |
| --- | --- | --- | --- |
| Name: | Print name | Name: | Print name |
| Signature: |  | Signature: |  |
| Date: | DD / MM / YYYY | Date: | DD / MM / YYYY |

# 8. Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| 01 | DD/MM/YYYY | Initial issue |  |  |
|  | DD/MM/YYYY |  |  |  |
|  | DD/MM/YYYY |  |  |  |