Shaft Alignment Report

| Document Ref: | Enter reference | Revision: | Enter revision |
| --- | --- | --- | --- |
| Project Name: | Enter project name | Contract No: | Enter contract number |
| Date: | DD / MM / YYYY | Aligned By: | Name and company |
| Witnessed By: | Name and role | Approved By: | Name and role |

# 1. Equipment Details

| Parameter | Driven Machine | Driver (Motor) |
| --- | --- | --- |
| Tag / Reference |  |  |
| Manufacturer |  |  |
| Model |  |  |
| Serial Number |  |  |
| Rated Speed (RPM) |  |  |
| Rated Power (kW) |  |  |
| Shaft Diameter (mm) |  |  |
| Bearing Type | e.g. Ball, roller, sleeve |  |

# 2. Coupling Details

| Coupling Type: | e.g. Flexible jaw, disc, gear, grid, spacer, rigid |
| --- | --- |
| Coupling Manufacturer: | Enter manufacturer name |
| Coupling Model / Size: | Enter model and size |
| Spacer Length (mm): | Enter spacer distance between shaft ends (if applicable) |
| Distance Between Measurement Points (mm): | Enter distance between sensor positions |

# 3. Alignment Method and Equipment

| Alignment Method: | Laser / Dial indicator (reverse indicator) / Straight edge |
| --- | --- |
| Alignment Tool Manufacturer: | e.g. Fixturlaser, Easy-Laser, Pruftechnik / Fluke |
| Tool Model: | Enter model number |
| Tool Serial Number: | Enter serial number |
| Calibration Date: | DD / MM / YYYY |
| Calibration Certificate Ref: | Enter certificate reference |

# 4. Pre-Alignment Checks

| No. | Check Item | Pass | Fail | N/A | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Foundation bolts checked and tightened to specification |  |  |  |  |
| 2 | Baseplate / sole plate level and grouted correctly |  |  |  |  |
| 3 | Soft foot check completed and corrected (max 0.05mm) |  |  |  |  |
| 4 | Pipe strain eliminated – flanges disconnected to check |  |  |  |  |
| 5 | Coupling bolts removed and coupling halves free to move |  |  |  |  |
| 6 | Shaft runout checked and within tolerance (max 0.025mm TIR) |  |  |  |  |
| 7 | Coupling hub runout checked and within tolerance |  |  |  |  |
| 8 | All shim packs clean, flat, and corrosion-free |  |  |  |  |
| 9 | Shim stack limited to maximum 5 shims per foot |  |  |  |  |
| 10 | Thermal growth compensation calculated and applied (if applicable) |  |  |  |  |
| 11 | Equipment at stable operating temperature (cold / warm alignment noted) |  |  |  |  |
| 12 | Alignment targets set per coupling manufacturer’s tolerances |  |  |  |  |

# 5. Alignment Tolerances

| Parameter | Coupling Tolerance | Excellent | Acceptable |
| --- | --- | --- | --- |
| Horizontal offset (mm) |  |  |  |
| Vertical offset (mm) |  |  |  |
| Horizontal angularity (mm/100mm) |  |  |  |
| Vertical angularity (mm/100mm) |  |  |  |

# 6. Alignment Results

## 6.1 Initial (As-Found) Alignment

| Parameter | Measured Value | Tolerance | Within Tolerance? |
| --- | --- | --- | --- |
| Horizontal offset (mm) |  |  | Y / N |
| Vertical offset (mm) |  |  | Y / N |
| Horizontal angularity (mm/100mm) |  |  | Y / N |
| Vertical angularity (mm/100mm) |  |  | Y / N |

## 6.2 Final (As-Left) Alignment

| Parameter | Measured Value | Tolerance | Within Tolerance? |
| --- | --- | --- | --- |
| Horizontal offset (mm) |  |  | Y / N |
| Vertical offset (mm) |  |  | Y / N |
| Horizontal angularity (mm/100mm) |  |  | Y / N |
| Vertical angularity (mm/100mm) |  |  | Y / N |

# 7. Correction Record

| Machine Moved: | Driver (motor) / Driven machine |
| --- | --- |
| Vertical Correction (shims): | Front feet: +/– mm Rear feet: +/– mm |
| Horizontal Correction: | Front feet: +/– mm Rear feet: +/– mm |
| Shim Material Used: | e.g. Stainless steel, pre-cut, laminated |
| Number of Shims Per Foot: | Front left: Front right: Rear left: Rear right: |
| Final Soft Foot Check: | mm (all feet must be < 0.05mm) |
| Foundation Bolts Re-Torqued: | Yes / No – torque value Nm |

# 8. Post-Alignment Verification

| No. | Check Item | Pass | Fail | N/A | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Final alignment values within specified tolerance |  |  |  |  |
| 2 | All foundation bolts re-torqued after alignment |  |  |  |  |
| 3 | Coupling bolts refitted and torqued to specification |  |  |  |  |
| 4 | Coupling guard refitted (before running) |  |  |  |  |
| 5 | Piping reconnected – no pipe strain reintroduced |  |  |  |  |
| 6 | Dowel pins installed (if required by specification) |  |  |  |  |
| 7 | Laser alignment report printed and attached |  |  |  |  |
| 8 | Alignment data saved to alignment tool memory (if applicable) |  |  |  |  |

# 9. Sign-Off

| Alignment Engineer |  | Witness / Inspector |  |
| --- | --- | --- | --- |
| Name: | Print name | Name: | Print name |
| Company: | Enter company | Position: | Enter role |
| Signature: |  | Signature: |  |
| Date: | DD / MM / YYYY | Date: | DD / MM / YYYY |

# 10. Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| 01 | DD/MM/YYYY | Initial issue |  |  |
|  | DD/MM/YYYY |  |  |  |
|  | DD/MM/YYYY |  |  |  |