Glare and Light Spill Assessment

| Document Ref: | Enter reference | Revision: | Enter revision |
| --- | --- | --- | --- |
| Project Name: | Enter project name | Contract No: | Enter contract number |
| Date: | DD / MM / YYYY | Assessed By: | Name and role |
| Approved By: | Name and role | Site Address: | Enter site address |

# 1. Assessment Details

| Purpose of Assessment: | e.g. Night works lighting, permanent site compound, tower crane lighting |
| --- | --- |
| ILP Environmental Zone: | E1 (intrinsically dark) / E2 (low district brightness) / E3 (medium) / E4 (high) |
| Hours of Illumination: | e.g. 18:00 – 07:00, dusk to dawn, 24/7 |
| Planning Condition Ref: | Enter any lighting-related planning conditions or N/A |
| Applicable Standards: | ILP Guidance Note GN01/21, CIE 150:2017, BS EN 12464-2, Clean Neighbourhoods and Environment Act 2005 |

# 2. Sensitive Receptors

| No. | Receptor Description | Address / Location | Distance (m) | Direction | Sensitivity |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  | m | e.g. N, SE | High / Med / Low |
| 2 |  |  | m | e.g. N, SE | High / Med / Low |
| 3 |  |  | m | e.g. N, SE | High / Med / Low |
| 4 |  |  | m | e.g. N, SE | High / Med / Low |
| 5 |  |  | m | e.g. N, SE | High / Med / Low |
| 6 |  |  | m | e.g. N, SE | High / Med / Low |
| 7 |  |  | m | e.g. N, SE | High / Med / Low |
| 8 |  |  | m | e.g. N, SE | High / Med / Low |

# 3. ILP Obtrusive Light Limits

The following maximum values apply to the Environmental Zone identified above, in accordance with ILP Guidance Note GN01/21.

| Parameter | E1 | E2 | E3 | E4 |
| --- | --- | --- | --- | --- |
| Light into windows (Ev, lux) – pre-curfew | 2 | 5 | 10 | 25 |
| Light into windows (Ev, lux) – post-curfew | 0 | 1 | 2 | 5 |
| Luminaire intensity (cd) – pre-curfew | 2,500 | 7,500 | 10,000 | 25,000 |
| Luminaire intensity (cd) – post-curfew | 0 | 500 | 1,000 | 2,500 |
| Upward light ratio (ULR) – maximum % | 0% | 2.5% | 5% | 15% |
| Building luminance (cd/m²) – pre-curfew | 0 | 5 | 10 | 25 |

# 4. Lighting Installation Details

| No. | Luminaire Location | Type | Wattage | Height (m) | Aiming Angle | Shielding | ULR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | e.g. LED flood | W | m | degrees | Y / N | % |
| 2 |  | e.g. LED flood | W | m | degrees | Y / N | % |
| 3 |  | e.g. LED flood | W | m | degrees | Y / N | % |
| 4 |  | e.g. LED flood | W | m | degrees | Y / N | % |
| 5 |  | e.g. LED flood | W | m | degrees | Y / N | % |
| 6 |  | e.g. LED flood | W | m | degrees | Y / N | % |
| 7 |  | e.g. LED flood | W | m | degrees | Y / N | % |
| 8 |  | e.g. LED flood | W | m | degrees | Y / N | % |
| 9 |  | e.g. LED flood | W | m | degrees | Y / N | % |
| 10 |  | e.g. LED flood | W | m | degrees | Y / N | % |

# 5. Light Spill Measurements / Predictions

| No. | Receptor | Predicted Ev (lux) | ILP Limit (lux) | Compliant? | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 |  | lux | lux | Yes / No |  |
| 2 |  | lux | lux | Yes / No |  |
| 3 |  | lux | lux | Yes / No |  |
| 4 |  | lux | lux | Yes / No |  |
| 5 |  | lux | lux | Yes / No |  |
| 6 |  | lux | lux | Yes / No |  |

# 6. Mitigation Measures

| No. | Control Measure | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | All luminaires aimed downwards (below horizontal plane) to minimise upward light |  |  |  |  |
| 2 | Luminaires fitted with cowls, baffles, or louvres to control spill |  |  |  |  |
| 3 | Asymmetric luminaires used where spill control is critical |  |  |  |  |
| 4 | Mounting heights minimised to reduce the spread of light beyond site boundary |  |  |  |  |
| 5 | Luminaires positioned to direct light away from sensitive receptors |  |  |  |  |
| 6 | Warm white colour temperature selected (3000K or below) to reduce sky glow |  |  |  |  |
| 7 | Timers or motion sensors installed to switch off lighting when not needed |  |  |  |  |
| 8 | Separate circuits provided for areas with different curfew requirements |  |  |  |  |
| 9 | Lighting switched off or reduced after curfew hours where practicable |  |  |  |  |
| 10 | Temporary hoarding or screening used to block direct light towards receptors |  |  |  |  |
| 11 | Tower crane aviation lights assessed for neighbour impact |  |  |  |  |
| 12 | Regular monitoring of light levels at receptor positions planned |  |  |  |  |

# 7. Ecological Considerations

| Bat Habitat Nearby? | Yes / No – if Yes, describe species and roost locations |
| --- | --- |
| Bat Mitigation Measures: | e.g. No lighting directed at bat corridors, amber LED filters, timing restrictions |
| Other Ecological Sensitivities: | e.g. Badger setts, nesting birds, watercourse corridors |
| Ecologist Consulted? | Yes / No – name and date |

# 8. Assessment Conclusion

Summarise the assessment findings: compliance with ILP guidance, adequacy of mitigation, any residual risks, and recommendations for ongoing monitoring.

# 9. Sign-Off

| Assessed By: | Name | Signature: |  |
| --- | --- | --- | --- |
| Position / Role: | Enter role | Date: | DD / MM / YYYY |
| Approved By: | Name | Signature: |  |
| Date: | DD / MM / YYYY | Next Review: | DD / MM / YYYY |

# 10. Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| 01 | DD/MM/YYYY | Initial issue |  |  |
|  | DD/MM/YYYY |  |  |  |
|  | DD/MM/YYYY |  |  |  |