Aeration System

Commissioning Record

| Document Ref: | Enter reference | Revision: | Enter revision |
| --- | --- | --- | --- |
| Project Name: | Enter project name | Contract No: | Enter contract number |
| Date: | DD / MM / YYYY | Commissioned By: | Name and company |
| Witnessed By: | Name and role | Approved By: | Name and role |

# 1. System Details

| System / Process: | e.g. Activated Sludge Plant (ASP), SBR, MBBR, MBR |
| --- | --- |
| Aeration Lane / Zone: | Enter lane reference and zone (anoxic, aerobic, swing) |
| Blower Tag(s): | Enter blower asset tag numbers |
| Blower Manufacturer: | Enter manufacturer name |
| Blower Type: | e.g. Positive displacement (Roots), turbo (high speed), screw |
| Blower Rated Capacity: | Nm³/hr per blower |
| Blower Motor Power: | kW per blower |
| Diffuser Type: | e.g. Fine bubble disc, tube, panel, membrane, coarse bubble |
| Diffuser Manufacturer: | Enter manufacturer name |
| Number of Diffusers: | Enter total per lane / zone |
| Design SOTR / SOTE: | kg O2/hr or % per metre depth |
| Design DO Setpoint: | mg/l |
| Drawing References: | Enter P&ID, air pipework layout, and diffuser layout drawing refs |

# 2. Pre-Commissioning Checks

| No. | Test / Check Item | Pass | Fail | N/A | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | All air pipework installed, supported, and pressure tested |  |  |  |  |
| 2 | Air pipework leak test completed and certificate available |  |  |  |  |
| 3 | Diffuser grid installed at correct level and orientation |  |  |  |  |
| 4 | All diffuser connections secure and airtight |  |  |  |  |
| 5 | Condensate drain pots installed and functional |  |  |  |  |
| 6 | Air isolation valves installed and labelled for each zone |  |  |  |  |
| 7 | Blower inlet filters clean and correctly fitted |  |  |  |  |
| 8 | Blower silencers (inlet and discharge) installed |  |  |  |  |
| 9 | Blower oil levels checked and lubrication completed |  |  |  |  |
| 10 | Blower vibration isolation mounts installed correctly |  |  |  |  |
| 11 | Anti-surge or blow-off valve installed and set (turbo blowers) |  |  |  |  |
| 12 | VSD installed and parameters configured for each blower |  |  |  |  |
| 13 | Dissolved oxygen (DO) probes installed, calibrated, and connected to SCADA |  |  |  |  |
| 14 | Air flow meters installed and calibrated |  |  |  |  |
| 15 | All electrical connections completed and tested |  |  |  |  |

# 3. Blower Commissioning Tests

| No. | Test / Check Item | Pass | Fail | N/A | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Blower rotation direction confirmed (bump test) |  |  |  |  |
| 2 | Blower started and run on no-load (discharge valve closed) |  |  |  |  |
| 3 | Discharge pressure verified at rated conditions |  |  |  |  |
| 4 | Air flow rate measured and compared to design duty |  |  |  |  |
| 5 | Motor current measured at various operating points |  |  |  |  |
| 6 | Bearing temperature monitored and within limits |  |  |  |  |
| 7 | Vibration levels measured and within manufacturer tolerance |  |  |  |  |
| 8 | Noise levels measured at 1m from blower housing |  |  |  |  |
| 9 | Inlet filter differential pressure within limits |  |  |  |  |
| 10 | Duty / standby changeover tested and verified |  |  |  |  |
| 11 | Auto speed control from DO signal verified |  |  |  |  |
| 12 | All alarms tested: high pressure, high temperature, vibration, surge |  |  |  |  |

# 4. Diffuser Performance Test

| No. | Test / Check Item | Pass | Fail | N/A | Initials |
| --- | --- | --- | --- | --- | --- |
| 1 | Lane filled to operating level before air introduced |  |  |  |  |
| 2 | Air distributed evenly across the diffuser grid (visual check) |  |  |  |  |
| 3 | No dead spots or areas with no bubble coverage |  |  |  |  |
| 4 | No large bubbles indicating damaged or unseated diffusers |  |  |  |  |
| 5 | Back pressure at diffuser header consistent with manufacturer data |  |  |  |  |
| 6 | Diffuser DWP (dynamic wet pressure) measured and within specification |  |  |  |  |
| 7 | Dissolved oxygen profile measured across the lane (uniform distribution) |  |  |  |  |
| 8 | DO control loop responding correctly to setpoint changes |  |  |  |  |
| 9 | Air flow modulation by zone operating correctly |  |  |  |  |

# 5. Test Measurements

| Parameter | Design Value | Measured Value | Pass / Fail |
| --- | --- | --- | --- |
| Blower discharge pressure (mbar) |  |  | P / F |
| Air flow rate (Nm³/hr) |  |  | P / F |
| Blower motor current (A) |  |  | P / F |
| Blower bearing temperature (°C) |  |  | P / F |
| Blower vibration (mm/s) |  |  | P / F |
| Diffuser back pressure (mbar) |  |  | P / F |
| Dissolved oxygen achieved (mg/l) |  |  | P / F |
| DO response time to setpoint change (min) |  |  | P / F |
| MLSS concentration (mg/l) |  |  | P / F |
| Lane water depth (m) |  |  | P / F |
| Air temperature at discharge (°C) |  |  | P / F |
| Total power consumption (kW) |  |  | P / F |

# 6. Snags and Actions

| No. | Issue | Action | Owner | Status |
| --- | --- | --- | --- | --- |
| 1 |  |  |  | Open / Closed |
| 2 |  |  |  | Open / Closed |
| 3 |  |  |  | Open / Closed |
| 4 |  |  |  | Open / Closed |
| 5 |  |  |  | Open / Closed |
| 6 |  |  |  | Open / Closed |

# 7. Sign-Off

| Commissioning Engineer |  | Witness / Client Rep |  |
| --- | --- | --- | --- |
| Name: | Print name | Name: | Print name |
| Company: | Enter company | Position: | Enter role |
| Signature: |  | Signature: |  |
| Date: | DD / MM / YYYY | Date: | DD / MM / YYYY |

# 8. Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| 01 | DD/MM/YYYY | Initial issue |  |  |
|  | DD/MM/YYYY |  |  |  |
|  | DD/MM/YYYY |  |  |  |