Scaffold Inspection Checklist

A detailed scaffold inspection checklist for use by competent scaffold inspectors (CISRS qualified). Covers all key inspection areas as required by the Work at Height Regulations 2005 (Schedule 7), NASC TG20:13, BS EN 12811, and SG4:15. Inspections must be carried out before first use, at intervals not exceeding 7 days, after adverse weather, and after any event affecting stability.

| Inspection Ref: |  | Date: |  |
| --- | --- | --- | --- |
| Project Name: |  | Scaffold Tag No: |  |
| Contract No: |  | Location / Elevation: |  |
| Inspected By: |  | CISRS Card No: |  |

1. Scaffold Details

| Scaffold Type (independent, birdcage, tower, etc.): |  |
| --- | --- |
| Scaffold Contractor: |  |
| Design Drawing / Ref No: |  |
| Duty Load Class (1-6 per TG20/BS EN 12811): |  |
| Maximum Working Platform Load: |  |
| Maximum Number of Loaded Lifts: |  |
| Height (m): |  |
| Reason for Inspection: |  |

2. Reason for Inspection

| Reason for Inspection | Tick | Reason for Inspection | Tick |
| --- | --- | --- | --- |
| Initial erection complete |  | After substantial alteration |  |
| 7-day periodic inspection |  | After adverse weather event |  |
| After any event affecting stability |  | Before handover to user |  |
| Following reported concern / defect |  | Pre-dismantling inspection |  |

3. Base and Foundations

| No. | Inspection Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Scaffold erected on firm, level ground or adequate foundation |  |  |  |  |
| 2 | Base plates are correctly positioned under all standards |  |  |  |  |
| 3 | Sole boards / sole plates are adequate for ground conditions |  |  |  |  |
| 4 | No standards are bearing directly on soft or uncompacted ground |  |  |  |  |
| 5 | Adjustable base jacks are within permitted extension range |  |  |  |  |

4. Structural Frame

| No. | Inspection Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Standards are plumb and correctly spaced per design / TG20 |  |  |  |  |
| 2 | Ledgers are level and correctly fixed to standards with right-angle couplers |  |  |  |  |
| 3 | Transoms are correctly positioned and secured |  |  |  |  |
| 4 | Bracing (ledger bracing and facade bracing) is installed per design |  |  |  |  |
| 5 | All couplers are correctly fitted and tightened (check torque where required) |  |  |  |  |
| 6 | No damaged, bent, or excessively corroded tubes in use |  |  |  |  |
| 7 | Tubes are of correct specification (48.3mm OD, 4mm wall minimum) |  |  |  |  |
| 8 | Joint pins or sleeve couplers are correctly positioned (not in same bay) |  |  |  |  |
| 9 | Lift heights comply with design requirements |  |  |  |  |

5. Working Platforms and Boards

| No. | Inspection Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Working platforms are fully boarded with no gaps exceeding 25mm |  |  |  |  |
| 2 | Boards are secured against displacement (board retaining clips or similar) |  |  |  |  |
| 3 | Boards are in good condition — not split, warped, or excessively worn |  |  |  |  |
| 4 | Platform width is adequate for intended use (minimum 600mm or as designed) |  |  |  |  |
| 5 | Trap doors / access hatches are functional and self-closing where fitted |  |  |  |  |
| 6 | No excessive deflection of boards under load |  |  |  |  |

6. Guard Rails and Edge Protection

| No. | Inspection Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Guard rails at 950mm minimum height on all open sides and ends |  |  |  |  |
| 2 | Mid-rails fitted between guard rail and toe board |  |  |  |  |
| 3 | Toe boards minimum 150mm height fitted on all open sides and ends |  |  |  |  |
| 4 | Brick guards / debris netting fitted where required |  |  |  |  |
| 5 | Sheeting / monarflex fitted where specified (wind loading checked) |  |  |  |  |
| 6 | Fan protection or crash deck fitted where public or workers below |  |  |  |  |
| 7 | Ladder access provided and secured at every lift used for access |  |  |  |  |

7. Ties and Restraints

| No. | Inspection Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Ties installed at correct centres per design / TG20 requirements |  |  |  |  |
| 2 | Tie types are as specified (through-tie, box-tie, reveal-tie, etc.) |  |  |  |  |
| 3 | All ties are secure and not displaced or loosened |  |  |  |  |
| 4 | Tie pattern is maintained through all lifts including top lift |  |  |  |  |
| 5 | Buttress or raker ties provided where wall ties cannot be installed |  |  |  |  |
| 6 | Tie removal protocol communicated — no ties removed without approval |  |  |  |  |

8. Access and Signage

| No. | Inspection Item | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Ladders are secured at top and bottom and extend 1m above landing |  |  |  |  |
| 2 | Internal ladder access is provided at suitable intervals |  |  |  |  |
| 3 | Stair tower access provided for scaffolds over 3 lifts (if specified) |  |  |  |  |
| 4 | Access gates at loading bays are self-closing and clearly marked |  |  |  |  |
| 5 | Scaffold tag / status board is displayed at all access points |  |  |  |  |
| 6 | Inspection signage showing current inspection status is visible |  |  |  |  |

9. Inspection Outcome

| Overall Result (Pass / Fail / Conditional): |  |
| --- | --- |
| Scaffold Status Tag Updated? (Green/Amber/Red): |  |
| Defects Requiring Immediate Action: |  |
| Next Inspection Due Date: |  |

10. Notes / Additional Comments

Record any defects, remedial actions, restrictions on use, or observations.

11. Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Scaffold Inspector (CISRS) |  |  |  |
| Scaffold Supervisor |  |  |  |
| Site Manager |  |  |  |

Document Control

| Rev | Date | Description | Author | Approved |
| --- | --- | --- | --- | --- |
| 01 |  |  |  |  |
| 02 |  |  |  |  |
| 03 |  |  |  |  |

References: Work at Height Regulations 2005 (Schedule 7); NASC TG20:13; SG4:15 (Preventing Falls in Scaffolding); BS EN 12811-1 (Scaffolding — Performance Requirements); CISRS Scheme.