Site Establishment Plan Checklist

Comprehensive mobilisation checklist for establishing a construction site in accordance with CDM 2015 and best practice

| Document Reference: | EST-CHK-001 |
| --- | --- |
| Revision: | 0 |
| Date: | DD/MM/YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Site Address: | Enter full site address |
| Mobilisation Date: | DD/MM/YYYY |
| Construction Start Date: | DD/MM/YYYY |
| Prepared By: | Enter name and role |
| Approved By: | Enter name and role |

Section A: Site Boundary, Hoarding, and Signage

| No. | Boundary / Hoarding Requirement | Yes | No | N/A | Owner / Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Site boundary defined and hoarding / fencing erected to full perimeter |  |  |  |  |
| 2 | Hoarding in good condition, minimum 2.4m height, stable and secure |  |  |  |  |
| 3 | Main site entrance gate installed with lock and signage |  |  |  |  |
| 4 | Pedestrian entrance separate from vehicle entrance where practicable |  |  |  |  |
| 5 | Site signage displayed at entrance: project name, PC details, emergency contacts, PPE requirements, prohibition signs |  |  |  |  |
| 6 | F10 notification displayed at site entrance (if notifiable project) |  |  |  |  |
| 7 | Considerate Constructors Scheme banner displayed (if registered) |  |  |  |  |
| 8 | Neighbours notified of works and contact details provided |  |  |  |  |

Section B: Site Compound and Welfare

| No. | Compound / Welfare Requirement | Yes | No | N/A | Owner / Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Site offices positioned and connected to power, water, data, and drainage |  |  |  |  |
| 2 | Site office accommodation adequate for project team size |  |  |  |  |
| 3 | Meeting room / briefing room available |  |  |  |  |
| 4 | Welfare facilities provided from Day 1 in accordance with CDM 2015 Schedule 2 |  |  |  |  |
| 5 | Toilets: adequate number for peak workforce (minimum 1 per 7 workers) |  |  |  |  |
| 6 | Washing facilities: hot and cold running water, soap, towels or dryers |  |  |  |  |
| 7 | Drying room provided for wet clothing |  |  |  |  |
| 8 | Canteen / rest area with seating, table, means of heating food, and drinking water |  |  |  |  |
| 9 | Changing facilities provided with lockers for personal clothing |  |  |  |  |
| 10 | First aid room or designated first aid area established with appropriate supplies |  |  |  |  |
| 11 | Site security office / gatehouse operational (if required) |  |  |  |  |
| 12 | CCTV installed and operational (if required) |  |  |  |  |
| 13 | Car parking areas designated for site personnel and visitors |  |  |  |  |

Section C: Temporary Services

| No. | Temporary Services Requirement | Yes | No | N/A | Owner / Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Temporary electrical supply installed by competent electrician and certificated |  |  |  |  |
| 2 | Temporary distribution boards installed, RCD protected, and labelled |  |  |  |  |
| 3 | Temporary water supply connected and tested |  |  |  |  |
| 4 | Temporary drainage / foul drainage connected or chemical toilets serviced |  |  |  |  |
| 5 | Temporary lighting installed for compound, access routes, and work areas |  |  |  |  |
| 6 | Data and telecommunications connected (broadband, phone lines) |  |  |  |  |
| 7 | Fire alarm system installed in offices and welfare units |  |  |  |  |
| 8 | Fire extinguishers provided in offices, canteen, and at fire points across site |  |  |  |  |
| 9 | Emergency lighting installed in enclosed temporary structures |  |  |  |  |

Section D: Access, Traffic, and Logistics

| No. | Access / Traffic Requirement | Yes | No | N/A | Owner / Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Vehicle access route from public highway defined and agreed with local authority |  |  |  |  |
| 2 | Temporary access road constructed to support delivery vehicles and plant |  |  |  |  |
| 3 | Wheel wash or road sweeping arrangements in place to prevent mud on public highway |  |  |  |  |
| 4 | Speed limit signs erected within site (typically 5–10 mph) |  |  |  |  |
| 5 | Pedestrian walkways defined, separated from vehicle routes, and clearly marked |  |  |  |  |
| 6 | Banksman arrangements in place for vehicle movements within site |  |  |  |  |
| 7 | Delivery management system established (booking-in system for HGVs) |  |  |  |  |
| 8 | Turning area provided for articulated vehicles |  |  |  |  |
| 9 | Disabled access provided to offices and welfare where required |  |  |  |  |

Section E: Environmental Controls

| No. | Environmental Requirement | Yes | No | N/A | Owner / Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Environmental constraints identified and briefed (ecology, watercourses, contamination, trees) |  |  |  |  |
| 2 | Tree protection fencing installed per BS 5837 at root protection area boundaries |  |  |  |  |
| 3 | Temporary drainage and silt control measures installed before earthworks commence |  |  |  |  |
| 4 | Spill kits positioned at fuel stores, plant refuelling points, and watercourse crossings |  |  |  |  |
| 5 | Waste storage area established with segregated, labelled containers |  |  |  |  |
| 6 | Dust and noise monitoring equipment installed where required by conditions or permits |  |  |  |  |
| 7 | Section 61 consent obtained (if applicable) before noisy works commence |  |  |  |  |

Section F: Health, Safety, and Administration

| No. | H&S / Admin Requirement | Yes | No | N/A | Owner / Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Construction Phase Plan prepared and available on site before works commence |  |  |  |  |
| 2 | Site rules document prepared and displayed |  |  |  |  |
| 3 | Site induction process established and first inductees processed |  |  |  |  |
| 4 | Induction room or area available with audio-visual equipment |  |  |  |  |
| 5 | Emergency evacuation plan prepared, assembly points identified and signed |  |  |  |  |
| 6 | Emergency contact list displayed in offices and welfare |  |  |  |  |
| 7 | First aiders appointed and names displayed |  |  |  |  |
| 8 | Accident book available in site office |  |  |  |  |
| 9 | Near miss reporting system in place (cards, app, or register) |  |  |  |  |
| 10 | Safety notice boards installed in prominent locations |  |  |  |  |
| 11 | PPE stocks available for issue (hard hats, hi-vis, gloves, glasses, boots) |  |  |  |  |
| 12 | Scaffold and temporary works register established |  |  |  |  |

Notes / Additional Comments

Enter any additional comments. Attach site establishment drawing showing compound layout, access routes, and laydown areas.

Approval

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Site Manager |  |  | DD/MM/YYYY |
| H&S Manager |  |  | DD/MM/YYYY |
| Project Manager |  |  | DD/MM/YYYY |
| Client (if required) |  |  | DD/MM/YYYY |

Document Control

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 0 | DD/MM/YYYY |  | Initial issue |