Formwork Inspection Checklist

A comprehensive pre-pour formwork and falsework inspection checklist covering foundations, vertical support, formwork face, safety provisions, and pre-pour readiness. Must be completed and signed off before any concrete pour is authorised. Compliant with BS 5975:2019, BS EN 12812, and CIRIA R108.

| Inspection Ref: |  | Date: |  |
| --- | --- | --- | --- |
| Project Name: |  | TW Register No: |  |
| Contract No: |  | Location / Pour Ref: |  |
| Inspected By: |  | Weather Conditions: |  |

1. Foundations and Base

Inspect the foundation and base support for the formwork/falsework system.

| No. | Inspection Item | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Foundations / sole plates are on firm, level, and adequate ground |  |  |  |  |
| 2 | Sole plates are of adequate size and in good condition |  |  |  |  |
| 3 | Base jacks are centred on sole plates and plumb |  |  |  |  |
| 4 | Ground bearing capacity is consistent with design assumptions |  |  |  |  |
| 5 | Adequate drainage provided to prevent water accumulation at base |  |  |  |  |

2. Vertical Support (Props / Standards)

Inspect all vertical support elements for compliance with the TW design.

| No. | Inspection Item | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Props / standards are vertical (plumb) and correctly spaced per design |  |  |  |  |
| 2 | Props are not damaged, bent, or corroded beyond acceptable limits |  |  |  |  |
| 3 | Correct prop type and capacity being used as per design |  |  |  |  |
| 4 | Extension of adjustable props is within manufacturer's limits |  |  |  |  |
| 5 | All pin connections are fully engaged and secured |  |  |  |  |
| 6 | Bracing is installed in accordance with the design drawing |  |  |  |  |
| 7 | Lacing and ledgers are fitted at correct heights |  |  |  |  |
| 8 | Props are not bearing on previously cast concrete unless approved |  |  |  |  |
| 9 | Re-propping from below is in place where required by design |  |  |  |  |

3. Formwork Face and Geometry

Inspect the formwork face for alignment, cleanliness, and readiness.

| No. | Inspection Item | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Formwork face is clean and free from debris |  |  |  |  |
| 2 | Release agent has been applied to all formwork faces |  |  |  |  |
| 3 | Formwork is correctly aligned to design dimensions and tolerances |  |  |  |  |
| 4 | Formwork joints are sealed to prevent grout loss |  |  |  |  |
| 5 | Kickers are correctly positioned and secured |  |  |  |  |
| 6 | Chamfer strips and drip details are fitted where specified |  |  |  |  |
| 7 | Box-outs, cast-in items, and sleeves are correctly positioned and secured |  |  |  |  |
| 8 | Access for concrete placement and vibration is adequate |  |  |  |  |
| 9 | Formwork ties are correctly installed and tightened |  |  |  |  |

4. Safety and Access

Confirm all safety provisions and access arrangements are in place.

| No. | Inspection Item | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Edge protection and guard rails are in place at all open edges |  |  |  |  |
| 2 | Safe access / egress is provided to the formwork (ladder, stair tower) |  |  |  |  |
| 3 | Working platform is complete with toe boards and guard rails |  |  |  |  |
| 4 | Exclusion zone is established below the formwork area |  |  |  |  |
| 5 | Permit to load has been issued by the TW Coordinator |  |  |  |  |
| 6 | All operatives briefed on pour sequence and emergency procedures |  |  |  |  |
| 7 | Concrete pump / skip access route is planned and clear |  |  |  |  |
| 8 | Vibrator and power supply are available and tested |  |  |  |  |
| 9 | Pour rate limitations are communicated to concrete supplier |  |  |  |  |

5. Pre-Pour Readiness

Final checks before authorising concrete placement.

| No. | Inspection Item | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Reinforcement inspection completed and accepted (hold point) |  |  |  |  |
| 2 | All cast-in services, ducts, and fixings checked and approved |  |  |  |  |
| 3 | Cover meters / spacers checked and adequate |  |  |  |  |
| 4 | Concrete specification confirmed with supplier (mix design, slump) |  |  |  |  |
| 5 | Sufficient concrete ordered for the pour volume |  |  |  |  |
| 6 | Contingency arrangements in place (pump breakdown, supply delay) |  |  |  |  |
| 7 | Weather forecast reviewed — no extreme conditions expected |  |  |  |  |
| 8 | TW Coordinator has issued permit to load / pour authorisation |  |  |  |  |

6. Inspection Outcome

| Inspection Result: |  |
| --- | --- |
| Conditions / Restrictions: |  |
| Re-Inspection Required? (Yes/No): |  |
| Re-Inspection Date (if applicable): |  |

7. Notes / Additional Comments

Record any defects observed, corrective actions required, or observations about formwork condition.

8. Sign-Off

| Role | Name | Signature | Date / Time |
| --- | --- | --- | --- |
| Formwork Inspector |  |  |  |
| TW Coordinator |  |  |  |
| Site Engineer |  |  |  |
| Supervisor / Foreman |  |  |  |

Document Control

| Rev | Date | Description | Author | Approved |
| --- | --- | --- | --- | --- |
| 01 |  |  |  |  |
| 02 |  |  |  |  |
| 03 |  |  |  |  |

References: BS 5975:2019; BS EN 12812 (Falsework); CIRIA R108 (Concrete Pressure on Formwork); CDM 2015; Concrete Society Technical Report 64.