Pedestrian Segregation Plan Checklist

For verifying that adequate pedestrian segregation measures are in place on construction sites in accordance with HSE guidance HSG144 and CDM 2015

| Document Reference: | PED-SEG- |
| --- | --- |
| Revision: | 0 |
| Inspection Date: | DD/MM/YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Inspector Name: | Enter name and role |

Section A: Pedestrian Routes

| No. | Pedestrian Route Requirement | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Dedicated pedestrian routes established throughout the site from entrance to all work areas and welfare |  |  |  |  |
| 2 | Pedestrian routes physically separated from vehicle routes using barriers, Heras fencing, concrete barriers, or armco |  |  |  |  |
| 3 | Separation maintained at all times; temporary removal of barriers requires risk assessment and banksman |  |  |  |  |
| 4 | Pedestrian routes clearly marked and signed (blue pedestrian route signs, directional arrows) |  |  |  |  |
| 5 | Routes minimum 1.0m width (1.5m where two-way traffic expected) |  |  |  |  |
| 6 | Routes maintained in good condition: level surface, no trip hazards, no standing water, no mud |  |  |  |  |
| 7 | Routes adequately lit for all working hours including winter months (minimum 20 lux, 50 lux preferred) |  |  |  |  |
| 8 | Tactile surfaces or contrast markings provided where pedestrians need to be alerted to hazards |  |  |  |  |

Section B: Crossing Points

| No. | Crossing Point Requirement | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Designated crossing points provided where pedestrian routes must cross vehicle routes |  |  |  |  |
| 2 | Crossing points clearly marked with painted lines, signage, and warning markings on road surface |  |  |  |  |
| 3 | Stop / Give Way signs installed for vehicles approaching pedestrian crossings |  |  |  |  |
| 4 | Pedestrian priority established at crossing points (vehicles must give way) |  |  |  |  |
| 5 | Physical measures installed at crossings: speed bumps, raised platforms, chicanes to slow vehicles |  |  |  |  |
| 6 | Banksman / traffic marshal deployed at high-traffic crossings during peak delivery hours |  |  |  |  |
| 7 | Audible and/or visual warnings at crossings where visibility is restricted |  |  |  |  |
| 8 | Crossing points positioned for maximum visibility; no blind corners or obstructed sight lines |  |  |  |  |

Section C: Exclusion Zones

| No. | Exclusion Zone Requirement | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Plant and vehicle operating areas clearly demarcated as exclusion zones for pedestrians |  |  |  |  |
| 2 | Exclusion zones barriered with Heras fencing, armco, or substantial barriers (not just tape) |  |  |  |  |
| 3 | Warning signs displayed at all entry points to exclusion zones |  |  |  |  |
| 4 | Pedestrians briefed during induction: no entry into plant exclusion zones under any circumstances |  |  |  |  |
| 5 | Banksman controls access where pedestrians must occasionally enter plant operating areas |  |  |  |  |
| 6 | Crane slew / lift radius zones barriered during operations |  |  |  |  |
| 7 | Loading / unloading areas designated as pedestrian-free zones during deliveries |  |  |  |  |

Section D: Public Interface

| No. | Public Interface Requirement | Yes | No | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Public footpaths and rights of way identified and maintained throughout the works |  |  |  |  |
| 2 | Temporary pedestrian diversions provided where existing footpaths are affected |  |  |  |  |
| 3 | Diversions clearly signed with advance warning, directional signs, and diversion route marked |  |  |  |  |
| 4 | Hoardings and barriers provided to protect the public from construction activities |  |  |  |  |
| 5 | Hoarding in good condition: minimum 2.4m height, stable, no gaps, maintained clean and tidy |  |  |  |  |
| 6 | Public interface points risk-assessed: vehicle crossings over footpaths, scaffold over footpaths, crane oversailing |  |  |  |  |
| 7 | Marshal / traffic management deployed where construction vehicles cross public footpaths |  |  |  |  |
| 8 | Vulnerable pedestrian groups considered: wheelchair users, pushchairs, visually impaired (DDA compliance) |  |  |  |  |

Actions Arising

| No. | Action Required | Responsible | Priority | Target Date | Status |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |

Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Inspector |  |  | DD/MM/YYYY |
| Site Manager |  |  | DD/MM/YYYY |

Document Control

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 0 | DD/MM/YYYY |  | Initial issue |