Chapter 8 Signing and Guarding Checklist

Inspection checklist for temporary traffic management on public highways in accordance with Chapter 8 of the Traffic Signs Manual and the TSRGD 2016

| Inspection Reference: | CH8-INS- |
| --- | --- |
| Date: | DD/MM/YYYY |
| Time: | HH:MM |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Road Name / Location: | Enter road name, section, and nearest landmark or grid ref |
| Road Speed Limit: | mph |
| Road Type: | Single carriageway / Dual carriageway / Motorway |
| TM Layout Drawing Ref: |  |
| Section 81 / NRSWA Permit Ref: |  |
| TM Contractor: |  |
| Inspector Name: | Enter name and role |

Section A: Signs

| No. | Signing Check | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Signing layout matches the approved traffic management drawing / Section 81 permit |  |  |  |  |
| 2 | All signs are Chapter 8 compliant (Traffic Signs Regulations and General Directions 2016) |  |  |  |  |
| 3 | Signs are correct size for the road speed limit (minimum 600mm for 30mph, 750mm for 40mph, 900mm for 50mph+) |  |  |  |  |
| 4 | Signs are clean, legible, undamaged, and retroreflective sheeting in good condition |  |  |  |  |
| 5 | Sign faces oriented correctly towards approaching traffic |  |  |  |  |
| 6 | Signs positioned at correct height: bottom edge minimum 900mm above carriageway level (or as per design) |  |  |  |  |
| 7 | Advance warning signs placed at correct distance from works (per Table 3.1 of Chapter 8) |  |  |  |  |
| 8 | Repeat signs installed where sight distances require them |  |  |  |  |
| 9 | Signs not obscured by vegetation, plant, materials, or other signs |  |  |  |  |
| 10 | All temporary signs removed or covered when not in use (especially at night if works are not in progress) |  |  |  |  |
| 11 | Contradictory permanent signs covered or removed during the works period |  |  |  |  |
| 12 | Sign frames and supports stable, weighted at base, and not a hazard to pedestrians or cyclists |  |  |  |  |

Section B: Cones and Tapers

| No. | Cone / Taper Check | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Cones placed at correct spacing for the road speed limit (per Table 4.1: e.g., 3m for 30mph, 6m for 40mph) |  |  |  |  |
| 2 | Cone size appropriate: minimum 450mm for roads up to 30mph, 750mm for roads over 30mph |  |  |  |  |
| 3 | Cones in good condition: not faded, cracked, or missing reflective sleeves |  |  |  |  |
| 4 | Taper length correct for the road speed limit and number of lanes closed (per Table 4.2) |  |  |  |  |
| 5 | Lead-in taper installed at the correct angle (1:road speed in mph for single lane closure) |  |  |  |  |
| 6 | Exit taper installed to guide traffic back to normal running lane |  |  |  |  |
| 7 | Cones placed in straight lines using string line or GPS for alignment |  |  |  |  |
| 8 | Longitudinal safety zone clear of all plant, materials, and personnel |  |  |  |  |
| 9 | Lateral safety zone maintained between live traffic and working space |  |  |  |  |

Section C: Barriers and Guarding

| No. | Guarding Check | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Barriers installed where pedestrians are affected by the works (pedestrian barriers / A-frames) |  |  |  |  |
| 2 | Barriers continuous, stable, and provide physical separation from live traffic |  |  |  |  |
| 3 | Pedestrian route maintained past the works: minimum 1.2m width, level surface, tactile paving where needed |  |  |  |  |
| 4 | Ramps provided where level differences occur on pedestrian routes (maximum gradient 1:12) |  |  |  |  |
| 5 | Barriers fitted with reflective markings for night-time visibility |  |  |  |  |
| 6 | Road plates installed correctly: pinned, non-slip surface, ramped edges, signed |  |  |  |  |

Section D: Lighting

| No. | Lighting Check | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Lamps operating on all signs, cones, and barriers during hours of darkness (where required) |  |  |  |  |
| 2 | Warning lights placed on the first and last cones of the taper and at intervals along the works |  |  |  |  |
| 3 | Flashing amber beacons operational on works vehicles and plant within the highway |  |  |  |  |
| 4 | Supplementary floodlighting provided for the working area if required |  |  |  |  |
| 5 | All lighting checked at dusk and maintained throughout the hours of darkness |  |  |  |  |

Section E: Personnel and Competence

| No. | Personnel Check | Pass | Fail | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | All operatives working on or near the highway hold valid Chapter 8 / NRSWA qualifications |  |  |  |  |
| 2 | High-visibility PPE worn: minimum EN ISO 20471 Class 3 upper body garment for work on roads over 30mph |  |  |  |  |
| 3 | Operatives briefed on the traffic management layout, escape routes, and emergency procedures |  |  |  |  |
| 4 | Lookout / stop-go operative deployed where required (for shuttle working) |  |  |  |  |
| 5 | Traffic management inspected at regular intervals during the shift and after any incident or adverse weather |  |  |  |  |

Inspection Outcome

| Outcome | Tick |
| --- | --- |
| COMPLIANT — TM layout matches drawing, all items satisfactory. |  |
| NON-COMPLIANT — Deficiencies identified. Corrective action required immediately. |  |

Corrective Actions

| No. | Deficiency | Action Required | Responsible | By When | Status |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |

Inspector Sign-Off

| Role | Name | Signature | Date / Time |
| --- | --- | --- | --- |
| TM Inspector |  |  | DD/MM/YYYY HH:MM |
| TM Supervisor / Foreman |  |  | DD/MM/YYYY HH:MM |
| Site Manager (if required) |  |  | DD/MM/YYYY HH:MM |

Document Control

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 0 | DD/MM/YYYY |  | Initial issue |