Concrete Pump Pre-Use Check Sheet

| Date: | DD/MM/YYYY | Project: | Enter project name |
| --- | --- | --- | --- |
| Pump Type: | Truck-mounted boom / Static line / Placing boom | Contract No: | Enter number |
| Make / Model: | Enter make and model | Fleet / Reg: | Enter fleet number or vehicle registration |
| Operator: | Enter operator name | CPCS Card: | Enter card number and expiry |
| Hire Company: | Enter hire company | Checked By: | Enter name |

Vehicle and General Checks

| No. | Check Item | OK | Defect | Action / Comments |
| --- | --- | --- | --- | --- |
| 1 | Vehicle road-worthiness – tyres, lights, mirrors, windscreen, wipers serviceable |  |  |  |
| 2 | Vehicle tax, MOT, and insurance current (check disc / DVLA online) |  |  |  |
| 3 | Fire extinguisher present, accessible, and in date |  |  |  |
| 4 | First aid kit present and stocked |  |  |  |
| 5 | Operator manual / quick reference card available in cab |  |  |  |
| 6 | LOLER thorough examination certificate in date (6-monthly for boom pumps) |  |  |  |
| 7 | PUWER inspection records available |  |  |  |
| 8 | All safety decals, labels, and load charts legible and in place |  |  |  |
| 9 | No visible damage, leaks, or defects to vehicle or pump bodywork |  |  |  |
| 10 | All guards and covers in place and secured |  |  |  |

Stabilisers / Outriggers

| No. | Check Item | OK | Defect | Action / Comments |
| --- | --- | --- | --- | --- |
| 1 | Outrigger legs and feet in good condition – no cracks, bending, or corrosion |  |  |  |
| 2 | Outrigger hydraulic rams operating correctly – smooth extension and retraction |  |  |  |
| 3 | Outrigger locking pins / safety locks operational |  |  |  |
| 4 | Outrigger pads / mats available and in good condition (timber or steel spreader plates) |  |  |  |
| 5 | Ground conditions assessed – firm, level, and capable of supporting outrigger loads |  |  |  |
| 6 | Outrigger positions do not encroach on excavations, drains, or buried services |  |  |  |
| 7 | Outrigger interlocks / sensors operational (prevents boom operation if outriggers not deployed) |  |  |  |

Boom and Pipework (Truck-Mounted Boom Pump)

| No. | Check Item | OK | Defect | Action / Comments |
| --- | --- | --- | --- | --- |
| 1 | Boom sections in good condition – no cracks, dents, or deformation |  |  |  |
| 2 | Boom pins and retaining clips in place and in good condition |  |  |  |
| 3 | Boom hydraulic rams – no leaks, smooth operation, no jerky movements |  |  |  |
| 4 | Boom hoses and couplings – no leaks, chafing, bulging, or damage |  |  |  |
| 5 | Delivery pipes and bends – no excessive wear, cracks, or blockages |  |  |  |
| 6 | Pipe clamps and safety chains / restraints in place and secure |  |  |  |
| 7 | End hose (flexible rubber) – in good condition, correct length, whip check fitted |  |  |  |
| 8 | Boom slew ring – smooth operation, no excessive play or grinding |  |  |  |
| 9 | Boom remote control unit – operational, emergency stop functional, batteries charged |  |  |  |
| 10 | Boom safety cut-out and overload protection operational |  |  |  |

Pump Unit and Hopper

| No. | Check Item | OK | Defect | Action / Comments |
| --- | --- | --- | --- | --- |
| 1 | Hopper – clean, no residual concrete build-up, grate in place and secured |  |  |  |
| 2 | Hopper safety grate interlock operational (pump stops if grate opened) |  |  |  |
| 3 | Pump cylinders – no leaks, smooth operation |  |  |  |
| 4 | Hydraulic system – oil level correct, no leaks from hoses, fittings, or rams |  |  |  |
| 5 | Water tank full (for priming, cleaning, and emergency washout) |  |  |  |
| 6 | Pressure gauge operational and reading within normal range |  |  |  |
| 7 | Emergency stop buttons operational (cab, hopper, remote) |  |  |  |
| 8 | Concrete delivery line clamps and couplings tight and in good condition |  |  |  |
| 9 | Agitator (if fitted) operational |  |  |  |
| 10 | Washout system / clean-out procedures understood and equipment available |  |  |  |

Site-Specific Checks

| No. | Check Item | OK | N/A | Action / Comments |
| --- | --- | --- | --- | --- |
| 1 | Pump location confirmed – firm level ground, away from excavations and overhead cables |  |  |  |
| 2 | Overhead power lines – confirmed clear of boom operating radius (minimum distances per HSE GS6) |  |  |  |
| 3 | Adequate space for concrete truck access and manoeuvring |  |  |  |
| 4 | Exclusion zone established around boom and delivery line |  |  |  |
| 5 | Traffic management in place for concrete truck deliveries |  |  |  |
| 6 | Washout area identified and containment in place (no discharge to drains or watercourses) |  |  |  |
| 7 | Communication plan agreed between pump operator, placing gang, and concrete truck drivers |  |  |  |
| 8 | Pour sequence and volume confirmed with site supervisor |  |  |  |

Check Outcome

| Pump Fit for Use: | Yes / No – if no, state reason and action taken |
| --- | --- |
| Defects Reported To: | Enter name and time reported |
| Defects Rectified Before Use: | Yes / No / N/A |

Sign-Off

| Role | Name | Signature | Date / Time |
| --- | --- | --- | --- |
| Pump Operator: |  |  |  |
| Site Supervisor: |  |  |  |