| SCAFFOLD INSPECTION RECORD |  |
| --- | --- |
| Document Ref: | Enter document reference |
| Revision: | Enter revision number |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Scaffold Reference / Tag No.: | Enter unique scaffold identification |
| Scaffold Location: | Describe location on site, elevation, zone |
| Scaffold Type: | Independent tied / Putlog / Birdcage / System / Mobile tower / Cantilever / Other |
| Scaffold Contractor: | Company name |
| Design Standard: | TG20 compliance / Bespoke design (state designer) |
| Maximum Working Load: | Enter duty classification: Light / General Purpose / Heavy / Special |
| Inspection Date: | DD / MM / YYYY |
| Inspection Time: | HH:MM |
| Inspected By: | Name, position, company and competence (e.g. CISRS scaffolder / CITB) |
| Weather Conditions: | Dry / Rain / Wind / Frost / Ice / Other |

This inspection record is completed in accordance with the Work at Height Regulations 2005 (Schedule 7), NASC guidance notes TG20 and SG4, and CDM 2015. Scaffolds must be inspected by a competent person before first use, at intervals not exceeding 7 days, and after any event likely to have affected stability.

1. Reason for Inspection

| Reason for Inspection (tick all that apply) | Tick | N/A |
| --- | --- | --- |
| Before first use after erection or substantial alteration |  |  |
| At intervals not exceeding 7 days since last inspection |  |  |
| After any event likely to have affected stability (high wind, heavy rain, impact) |  |  |
| Following a complaint or reported concern |  |  |
| Other (specify): |  |  |

2. Foundations and Base

| No. | Foundations and Base | Pass | Fail | Action / Comments |
| --- | --- | --- | --- | --- |
| 1 | Base plates and sole boards in position on firm, level ground |  |  |  |
| 2 | Standards plumb and correctly spaced |  |  |  |
| 3 | Standards continuous (joints staggered) and correctly coupled |  |  |  |
| 4 | Adequate base plate to sole board bearing area |  |  |  |
| 5 | Foundation adequate for imposed loads and ground conditions |  |  |  |

3. Standards, Ledgers, Transoms and Bracing

| No. | Standards, Ledgers, Transoms and Bracing | Pass | Fail | Action / Comments |
| --- | --- | --- | --- | --- |
| 6 | Ledgers level and correctly coupled to standards |  |  |  |
| 7 | Transoms / putlogs correctly positioned and secured |  |  |  |
| 8 | Bracing installed as per design (ledger bracing, plan bracing, facade bracing) |  |  |  |
| 9 | All couplers correctly fitted, tightened and of correct type (right-angle, swivel, putlog) |  |  |  |
| 10 | No damaged, corroded or bent tubes in use |  |  |  |
| 11 | Joints in standards and ledgers correctly positioned (not in same bay/lift) |  |  |  |

4. Ties and Anchors

| No. | Ties and Anchors | Pass | Fail | Action / Comments |
| --- | --- | --- | --- | --- |
| 12 | Scaffold ties installed at correct centres as per design / TG20 |  |  |  |
| 13 | All ties secure and load-bearing (not relying on push-pull alone) |  |  |  |
| 14 | Reveal ties wedged correctly (if used) |  |  |  |
| 15 | Through ties / anchor ties secure and in good condition |  |  |  |
| 16 | Tie pattern not compromised by removal for access or loading |  |  |  |
| 17 | Tie integrity not affected by adjacent works (demolition, cladding removal) |  |  |  |

5. Platforms and Boarding

| No. | Platforms and Boarding | Pass | Fail | Action / Comments |
| --- | --- | --- | --- | --- |
| 18 | Boarded lifts fully decked with no gaps exceeding 25mm |  |  |  |
| 19 | Boards in good condition, not split, warped or excessively worn |  |  |  |
| 20 | Boards properly supported (max overhang 150mm, min 4 supports per board if >3.9m) |  |  |  |
| 21 | Trap doors and access points properly fitted and operable |  |  |  |
| 22 | Platform width adequate for work and materials (min 600mm general, 800mm preferred) |  |  |  |
| 23 | Hop-up brackets secure and boards in place (if applicable) |  |  |  |

6. Guard Rails, Toe Boards and Edge Protection

| No. | Guard Rails, Toe Boards and Edge Protection | Pass | Fail | Action / Comments |
| --- | --- | --- | --- | --- |
| 24 | Top guard rail in place at all working lifts (minimum 950mm height) |  |  |  |
| 25 | Mid rail in place (or gap between top rail and toe board not exceeding 470mm) |  |  |  |
| 26 | Toe boards in place at all working lifts (minimum 150mm height) |  |  |  |
| 27 | Brick guards / debris netting installed where risk of falling objects |  |  |  |
| 28 | End protection in place at all open ends of platforms |  |  |  |
| 29 | Guard rails and toe boards at all internal edges adjacent to openings |  |  |  |
| 30 | Sheeting / monofilament securely fixed and not creating wind-loading issue |  |  |  |

7. Access

| No. | Access | Pass | Fail | Action / Comments |
| --- | --- | --- | --- | --- |
| 31 | Ladder access provided and correctly positioned (75° angle, extending 1m above landing) |  |  |  |
| 32 | Ladder secured at top and bottom |  |  |  |
| 33 | Internal staircase / ladder system in good condition (if fitted) |  |  |  |
| 34 | Landing platforms provided at suitable intervals |  |  |  |
| 35 | Access gates (if fitted) self-closing and operational |  |  |  |
| 36 | No unauthorised access possible to incomplete scaffold |  |  |  |

8. General Condition and Loading

| No. | General Condition and Loading | Pass | Fail | Action / Comments |
| --- | --- | --- | --- | --- |
| 37 | Scaffold tag / notice displayed and up to date (green = safe, red = unsafe) |  |  |  |
| 38 | Loading notice displayed stating maximum permitted loads |  |  |  |
| 39 | Scaffold not overloaded (materials, debris, snow, ice build-up) |  |  |  |
| 40 | No unauthorised alterations to scaffold since last inspection |  |  |  |
| 41 | Scaffold clear of debris and materials stored safely |  |  |  |
| 42 | Electrical safety — minimum clearance from overhead power lines maintained |  |  |  |
| 43 | Protection to public / third parties (fans, covered walkways, crash decks) in place |  |  |  |

9. Corrective Actions

Record any deficiencies and corrective actions required.

| No. | Item Ref | Corrective Action Required | By Whom | Completed Date |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

10. Overall Assessment and Scaffold Status

| Scaffold Status: | SAFE TO USE (Green Tag) / UNSAFE — DO NOT USE (Red Tag) / REQUIRES REMEDIAL ACTION |
| --- | --- |
| Tag Colour Applied: | Green / Red / Amber (if site uses three-colour system) |
| Inspector Comments: | Enter any observations |

11. Sign-Off

By signing below, the inspector confirms this inspection has been carried out diligently.

| Role | Print Name | Signature | Date |
| --- | --- | --- | --- |
| Scaffold Inspector (Competent Person) |  |  |  |
| Scaffold Contractor Supervisor |  |  |  |
| Site Manager / Agent |  |  |  |

12. Notes / Additional Comments

Enter any additional notes here

13. Document Control

| Rev | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |

References: WAH Regulations 2005, CDM 2015, NASC TG20:21, NASC SG4:22, BS EN 12811, BS EN 12810, PUWER 1998