Temporary Works Removal Record

Permit to remove and removal record for temporary works. No temporary works may be struck, dismantled, or altered without formal authorisation from the TWC. This permit confirms that the permanent works are self-supporting and the temporary works can be safely removed in the specified sequence. In accordance with BS 5975:2019 Clause 6.8 and BS EN 13670 (Execution of concrete structures) for formwork striking times.

Permit Details

| Permit No.: | Enter unique permit to remove number |
| --- | --- |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| TW Register Ref: | Enter temporary works register item number |
| Permit to Load Ref: | Enter associated permit to load number |

Temporary Works Details

| Temporary Works Title: | Enter descriptive title |
| --- | --- |
| TW Category (BS 5975): | Cat 1 / Cat 2 / Cat 3 |
| Location: | Enter precise location on site |
| Date Installed: | DD / MM / YYYY |
| Date Loaded: | DD / MM / YYYY |
| Permanent Works Supported: | Describe what permanent structure the TW was supporting |
| Removal Method Statement Ref: | Enter approved method statement for striking / removal |
| Removal Sequence Drawing Ref: | Enter drawing reference if applicable |

Concrete Strength Verification (If Applicable)

| Required Concrete Strength for Striking: | Enter minimum strength (N/mm²) per design or BS EN 13670 |
| --- | --- |
| Cube / Cylinder Test Reference: | Enter test specimen reference(s) |
| Actual Strength Achieved (N/mm²): | Enter test results |
| Age of Concrete at Striking (days): | Enter days since pour |
| Curing Conditions: | Describe curing method and ambient temperatures during curing period |
| Strength Requirement Met: | Yes / No — must be Yes before permit is issued |

Pre-Removal Inspection Checklist

All items must be Y or NA before the permit to remove can be issued. Any item marked N must be resolved first.

| No. | Pre-Removal Check Item | Y / N / NA | Comments |
| --- | --- | --- | --- |
| 1 | Permanent works have achieved adequate strength to be self-supporting (concrete cube results confirm minimum strength achieved) |  |  |
| 2 | Minimum striking time has elapsed per BS EN 13670 / BS 8110 or as specified by the permanent works designer |  |  |
| 3 | Concrete cube test results received and confirm adequate strength for the applied loads |  |  |
| 4 | Permanent works designer has confirmed the structure is self-supporting (written confirmation received) |  |  |
| 5 | No additional loads have been placed on the permanent works beyond those assumed in the striking assessment |  |  |
| 6 | Backpropping requirements have been assessed and implemented where required (multi-storey structures) |  |  |
| 7 | Removal sequence has been agreed and is documented in the method statement |  |  |
| 8 | Removal team has been briefed on the sequence, hazards, and emergency procedures |  |  |
| 9 | Adjacent temporary works that may be affected by the removal have been assessed |  |  |
| 10 | Exclusion zone established beneath and around the area of removal |  |  |
| 11 | Monitoring points established on the permanent works to detect any excessive deflection during striking |  |  |
| 12 | Weather conditions assessed — no extreme temperatures that could affect concrete strength assumptions |  |  |
| 13 | Props, falsework, or formwork will be eased gradually and uniformly — not suddenly released |  |  |
| 14 | Formwork striking will proceed in the correct sequence (sides before soffit, midspan before supports) |  |  |
| 15 | Adequate cranage or manual handling arrangements in place for removing TW components safely |  |  |
| 16 | Storage area identified for removed TW components |  |  |
| 17 | Risk assessment reviewed and all control measures in place |  |  |

Permit to Remove — Declaration

I, the Temporary Works Coordinator, hereby confirm that the permanent works are adequately self-supporting, the minimum striking criteria have been met, and it is safe to remove the temporary works identified above in accordance with the approved removal sequence. This permit authorises removal to commence.

Sign-Off — Permit to Remove

| Role | Name | Signature | Date / Time |
| --- | --- | --- | --- |
| Temporary Works Coordinator (TWC) |  |  | DD / MM / YYYY at HH:MM |
| Construction Manager / Site Agent |  |  | DD / MM / YYYY at HH:MM |
| Foreman / Ganger (receipt of permit) |  |  | DD / MM / YYYY at HH:MM |
| Permanent Works Designer (if confirmation required) |  |  | DD / MM / YYYY at HH:MM |

Post-Removal Confirmation

Complete this section after the temporary works have been fully removed.

| Date Removal Completed: | DD / MM / YYYY |
| --- | --- |
| Removal Completed By: | Enter foreman / supervisor name |
| Permanent Works Inspected After Striking: | Yes / No — confirm no cracking, excessive deflection, or damage |
| Deflection Measured After Striking: | Enter measurement if monitoring was specified |
| Any Damage to Permanent Works: | Yes / No — if yes, describe and raise NCR |
| TW Components Removed from Site: | Yes / No — confirm all materials cleared |
| TW Register Updated: | Yes / No — confirm register status changed to Closed |

Post-Removal Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Temporary Works Coordinator (TWC) |  |  | DD / MM / YYYY |
| Site Engineer / Construction Manager |  |  | DD / MM / YYYY |

Notes / Additional Comments

Enter any additional notes, cube test certificate references, deflection survey data, or backpropping arrangement details here.

Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| A |  | Initial issue |  |  |