Chlorination and Dechlorination Record

Record of pipeline chlorination, dechlorination, and bacteriological sampling for potable water mains and treated water systems. Completed as a permanent quality record and adoption / commissioning document. In accordance with BS EN 805 (Water supply — Requirements for systems outside buildings), WRAS Water Regulations, Water Supply (Water Quality) Regulations 2016, and the water company specification.

Project Details

| Document Ref: | Enter document reference |
| --- | --- |
| Revision: | Enter revision number |
| Certificate No.: | Enter unique certificate number |
| Date: | DD / MM / YYYY |
| Project Name: | Enter project name |
| Contract Number: | Enter contract number |
| Water Company / Authority: | Enter adopting water company name |
| Chlorination Contractor: | Enter specialist contractor name |
| Prepared By: | Name / role |
| Approved By: | Name / role |

Pipeline Details

| Pipeline / Main Reference: | Enter pipeline or water main designation |
| --- | --- |
| Section Description: | Enter section description (from — to, including chainage if applicable) |
| Drawing Reference: | Enter as-laid or GA drawing reference |
| Pipe Material: | e.g. DI, PE, uPVC, steel, GRP |
| Pipe Diameter (mm): | Enter nominal diameter |
| Total Length (m): | Enter total length of section to be chlorinated |
| Approximate Volume (litres): | Enter calculated pipeline volume |
| Service Type: | e.g. potable water supply, raw water, treated water |

Chlorination Details

| Chlorination Method: | e.g. sodium hypochlorite dosing, calcium hypochlorite tablets, chlorine gas |
| --- | --- |
| Chlorination Procedure Ref: | Enter approved procedure / method statement reference |
| Target Initial Chlorine Dose (mg/l): | Enter initial free chlorine concentration (typically 20-50 mg/l per BS EN 805) |
| Actual Initial Chlorine Dose (mg/l): | Enter measured initial concentration |
| Contact Period Required: | Enter required contact time (typically 16-24 hours per BS EN 805 / WRAS) |
| Chlorination Start Date / Time: | DD / MM / YYYY at HH:MM |
| Chlorination End Date / Time: | DD / MM / YYYY at HH:MM |
| Actual Contact Period: | Enter actual duration of chlorine contact |
| Residual Chlorine at End of Contact (mg/l): | Enter measured free chlorine residual (must be >0 mg/l at all sample points) |
| Water Temperature During Contact (deg C): | Enter water temperature |
| All Air Valves / Washouts Operated: | Yes / No — confirm all high and low points flushed |

Chlorine Residual Sample Points

| Sample Point 1 — Location: | Enter location and free chlorine residual (mg/l) |
| --- | --- |
| Sample Point 2 — Location: | Enter location and free chlorine residual (mg/l) |
| Sample Point 3 — Location: | Enter location and free chlorine residual (mg/l) |
| Sample Point 4 — Location: | Enter location and free chlorine residual (mg/l) |
| Minimum Residual Achieved (mg/l): | Enter lowest residual recorded across all sample points |
| Chlorination Result: | Pass / Fail (pass = residual chlorine >0 mg/l at all points after contact period) |

Dechlorination Details

| Dechlorination Method: | e.g. sodium thiosulphate dosing, ascorbic acid, activated carbon filter, natural dissipation |
| --- | --- |
| Dechlorination Chemical Used: | Enter chemical name and concentration |
| Volume of Dechlorination Agent (litres): | Enter volume dosed |
| Dechlorination Start Date / Time: | DD / MM / YYYY at HH:MM |
| Dechlorination Completion Date / Time: | DD / MM / YYYY at HH:MM |
| Residual Chlorine After Dechlorination (mg/l): | Enter confirmed zero or <0.1 mg/l reading |
| Discharge Point: | Enter discharge location |
| Environmental Permit / Consent Ref: | Enter consent reference for discharge to watercourse if applicable |
| Receiving Watercourse (if applicable): | Enter watercourse name |
| Dechlorination Result: | Pass / Fail (pass = chlorine residual at discharge ≤ permitted level) |

Bacteriological Sampling Results

Bacteriological samples must be taken after flushing and before the main is connected to the live network. Samples must be analysed by a UKAS-accredited laboratory.

| Sampling Date / Time: | DD / MM / YYYY at HH:MM |
| --- | --- |
| Sampling Points: | Enter locations where bacteriological samples taken |
| Number of Samples Taken: | Enter total number |
| Laboratory Name: | Enter UKAS-accredited laboratory name |
| Laboratory Reference: | Enter lab report reference |
| Coliforms (per 100ml): | Enter result (must be 0 per 100ml) |
| E. coli (per 100ml): | Enter result (must be 0 per 100ml) |
| Colony Count at 22 deg C (per ml): | Enter result |
| Colony Count at 37 deg C (per ml): | Enter result |
| Turbidity (NTU): | Enter result |
| Bacteriological Result: | Pass / Fail — state whether results meet Water Supply (Water Quality) Regulations |
| Re-Chlorination Required: | Yes / No — if bacteriological failure, describe remedial action |

Declaration

We hereby certify that the above pipeline section has been chlorinated, dechlorinated, and sampled in accordance with the approved procedure, BS EN 805, and the water company requirements. The bacteriological results confirm the pipeline is fit for connection to the potable water supply network.

Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Chlorination Operative |  |  | DD / MM / YYYY |
| Site Supervisor / Foreman |  |  | DD / MM / YYYY |
| Site Engineer (Main Contractor) |  |  | DD / MM / YYYY |
| Water Company Inspector / Representative |  |  | DD / MM / YYYY |

Notes / Additional Comments

Enter any additional notes, laboratory certificate references, COSHH assessment references for chlorination chemicals, or re-chlorination details here.

Document Control

| Revision | Date | Description of Change | Author | Approved By |
| --- | --- | --- | --- | --- |
| A |  | Initial issue |  |  |